1	gesis® CON - Building installation systems	2

1.1	3-pole - GST18i3	2

1.1.1	Connectors	2

1.1.1.1	Female connector with screw connection	2

1.1.1.2	Female connector with spring clamp connection	4

1.1.1.3	Male with screw connection	6

1.1.1.4	Male with spring clamp connection	8

1.1.2	Snap-In	10

1.1.2.1	Female connector with screw connection	10

1.1.2.2	Female connector with spring clamp connection	11

1.1.2.3	Male with screw connection	14

1.1.2.4	Male with spring clamp connection	15

1.1.3	Luminaire connection	18

1.1.4	Distribution blocks	19

1.1.5	Accessories	25

1.1.6	Cables	29

1.1.6.1	Cable assemblies, 1.5 mm², containing halogen	29

1.1.6.2	Cable assemblies, 1.5 mm², halogen-free	34

1.1.6.3	Cable assemblies, 1.5 mm², containing halogen, 90 °C	39

1.1.6.4	Cable assemblies, 2.5 mm², containing halogen	42

1.1.6.5	Cable assemblies, 2.5 mm², halogen-free	46

1.1.6.6	Cable assemblies, 2.5 mm², containing halogen, 90 °C	51

1.1.7	Outlets	54

1.1.8	Assembled Cables	56

1.2	4-pole - GST18i4	57

1.2.1	Connectors	57

1.2.1.1	Female connector with screw connection	57

1.2.1.2	Female connector with spring clamp connection	59

1.2.1.3	Male with screw connection	61

1.2.1.4	Male with spring clamp connection	63

1.2.2	Snap-In	65

1.2.2.1	Female connector with spring clamp connection	65

1.2.2.2	Male with spring clamp connection	67

1.2.3	Distribution blocks	68

1.2.4	Accessories	70

1.2.5	Cables	72

1.2.5.1	Cable assemblies, 1.5 mm², containing halogen	72

1.2.5.2	Cable assemblies, 1.5 mm², halogen-free	76

1.2.5.3	Cable assemblies, 2.5 mm², containing halogen	80

1.2.5.4	Cable assemblies, 2.5 mm², halogen-free	83

1.3	5-pole - GST18i5	87

1.3.1	Connectors	87

1.3.1.1	Female connector with screw connection	87

1.3.1.2	Male with screw connection	89

1.3.2	Snap-In	91

1.3.2.1	Female connector with spring clamp connection	91

1.3.2.2	Male with spring clamp connection	93

1.3.3	Luminaire connection	95

1.3.4	Distribution blocks	96

1.3.5	Accessories	103

1.3.6	Cables	105

1.3.6.1	Cable assemblies, 1.5 mm², containing halogen	105

1.3.6.2	Cable assemblies, 1.5 mm², halogen-free	109

1.3.6.3	Cable assemblies, 2.5 mm², containing halogen	113

1.3.6.4	Cable assemblies, 2.5 mm², halogen-free	115

1.4	6-pole - GST18i6	118

1.4.1	Connectors	118

1.4.1.1	Female connector with screw connection	118

1.4.1.2	Male with screw connection	119

1.4.2	Accessories	120

1.4.3	Cables	121

1.4.3.1	Cable assemblies, 1.5 mm², halogen-free	121

- gesis® CON - Building installation systems

  - 3-pole - GST18i3

    - Connectors

      - Female connector with screw connection

gesis® connector, female, 3-pole, 250 V, 20 A, white

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 3-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 6.5 - 10.5 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: N, PE, L

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 S B1 Z R1 WS

Part No.: 92.931.3053.0

gesis® connector, female, 3-pole, 250 V, 20 A, black

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 3-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 6.5 - 10.5 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: N, PE, L

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 S B1 Z R1 SW

Part No.: 92.931.3053.1

gesis® connector, female, 3-pole, 250 V, 20 A, pebble gray

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 3-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 6.5 - 10.5 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: N, PE, L

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 S B1 Z R1 GR01

Part No.: 92.931.3453.0

gesis® connector, female, 3-pole, 250 V, 20 A, light red

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 3-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 3, color coding light red.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 6.5 - 10.5 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: N, PE, L

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 S B1 Z R1 RT03

Part No.: 92.931.3653.0

gesis® connector, female, 3-pole, 250 V, 20 A, brown

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 3-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 4, color coding brown.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 6.5 - 10.5 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: 3, 2, 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 S B1 Z R1 BR05

Part No.: 92.931.3853.0

      - Female connector with spring clamp connection

gesis® connector, female, 3-pole, 250 V, 20 A, white

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 3-pole, turned contacts with spring clamp connection, compact housing with strain relief, field assembly, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 1.5 - 2.5 mm²

Terminal cross-section, stranded: 1.5 mm² with ferrule

Cable diameter: 7.8 - 10.3 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 2

Marking: N, PE, L

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F B2 Z R1 WS

Part No.: 92.933.0053.0

gesis® connector, female, 3-pole, 250 V, 20 A, black

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 3-pole, turned contacts with spring clamp connection, compact housing with strain relief, field assembly, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 1.5 - 2.5 mm²

Terminal cross-section, stranded: 1.5 mm² with ferrule

Cable diameter: 7.8 - 10.3 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 2

Marking: N, PE, L

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F B2 Z R1 SW

Part No.: 92.933.0053.1

gesis® connector, female, 3-pole, 250 V, 20 A, pebble gray

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 3-pole, turned contacts with spring clamp connection, compact housing with strain relief, field assembly, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 1.5 - 2.5 mm²

Terminal cross-section, stranded: 1.5 mm² with ferrule

Cable diameter: 7.8 - 10.3 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 2

Marking: N, PE, L

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F B2 Z R1 GR01

Part No.: 92.933.0153.0

gesis® connector, female, 3-pole, 250 V, 20 A, light red

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 3-pole, turned contacts with spring clamp connection, compact housing with strain relief, field assembly, mechanical coding code 3, color coding light red.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 1.5 - 2.5 mm²

Terminal cross-section, stranded: 1.5 mm² with ferrule

Cable diameter: 7.8 - 10.3 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 2

Marking: N, PE, L

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F B2 Z R1 RT03

Part No.: 92.933.0553.0

gesis® connector, female, 3-pole, 250 V, 20 A, brown

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 3-pole, turned contacts with spring clamp connection, compact housing with strain relief, field assembly, mechanical coding code 4, color coding brown.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 1.5 - 2.5 mm²

Terminal cross-section, stranded: 1.5 mm² with ferrule

Cable diameter: 7.8 - 10.3 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 2

Marking: 3, 2, 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F B2 Z R1 BR05

Part No.: 92.933.0453.0

      - Male with screw connection

gesis® connector, male, 3-pole, 250 V, 20 A, white

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 3-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 6.5 - 10.5 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: L, PE, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 S S1 Z R1 WS

Part No.: 92.932.3053.0

gesis® connector, male, 3-pole, 250 V, 20 A, black

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 3-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 6.5 - 10.5 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: L, PE, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 S S1 Z R1 SW

Part No.: 92.932.3053.1

gesis® connector, male, 3-pole, 250 V, 20 A, pebble gray

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 3-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 6.5 - 10.5 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: L, PE, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 S S1 Z R1 GR01

Part No.: 92.932.3453.0

gesis® connector, male, 3-pole, 250 V, 20 A, light red

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 3-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 3, color coding light red.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 6.5 - 10.5 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: L, PE, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 S S1 Z R1 RT03

Part No.: 92.932.3653.0

gesis® connector, male, 3-pole, 250 V, 20 A, brown

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 3-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 4, color coding brown.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 6.5 - 10.5 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: 1, 2, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 S S1 Z R1 BR05

Part No.: 92.932.3853.0

      - Male with spring clamp connection

gesis® connector, male, 3-pole, 250 V, 20 A, white

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 3-pole, turned contacts with spring clamp connection, compact housing with strain relief, field assembly, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 1.5 - 2.5 mm²

Terminal cross-section, stranded: 1.5 mm² with ferrule

Cable diameter: 7.8 - 10.3 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 2

Marking: L, PE, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F S2 Z R1 WS

Part No.: 92.934.0053.0

gesis® connector, male, 3-pole, 250 V, 20 A, black

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 3-pole, turned contacts with spring clamp connection, compact housing with strain relief, field assembly, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 1.5 - 2.5 mm²

Terminal cross-section, stranded: 1.5 mm² with ferrule

Cable diameter: 7.8 - 10.3 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 2

Marking: L, PE, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F S2 Z R1 SW

Part No.: 92.934.0053.1

gesis® connector, male, 3-pole, 250 V, 20 A, pebble gray

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 3-pole, turned contacts with spring clamp connection, compact housing with strain relief, field assembly, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 1.5 - 2.5 mm²

Terminal cross-section, stranded: 1.5 mm² with ferrule

Cable diameter: 7.8 - 10.3 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 2

Marking: L, PE, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F S2 Z R1 GR01

Part No.: 92.934.0153.0

gesis® connector, male, 3-pole, 250 V, 20 A, light red

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 3-pole, turned contacts with spring clamp connection, compact housing with strain relief, field assembly, mechanical coding code 3, color coding light red.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 1.5 - 2.5 mm²

Terminal cross-section, stranded: 1.5 mm² with ferrule

Cable diameter: 7.8 - 10.3 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 2

Marking: L, PE, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F S2 Z R1 RT03

Part No.: 92.934.0553.0

gesis® connector, male, 3-pole, 250 V, 20 A, brown

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 3-pole, turned contacts with spring clamp connection, compact housing with strain relief, field assembly, mechanical coding code 4, color coding brown.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 1.5 - 2.5 mm²

Terminal cross-section, stranded: 1.5 mm² with ferrule

Cable diameter: 7.8 - 10.3 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 2

Marking: 3, 2, 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F S2 Z R1 BR05

Part No.: 92.934.0453.0

    - Snap-In

      - Female connector with screw connection

gesis® Snap-In, female, 3-pole, for panel mounting, 250 V, 20 A, white

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 3-pole, turned contacts with screw connection, snapable in housing, with integrated locking, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 2.5 mm²

Housing material: Polyamide

Connections per pole: 1

Marking: N, PE, L

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 S B1 RV WS

Part No.: 92.031.8758.0

gesis® Snap-In, female, 3-pole, for panel mounting, 250 V, 20 A, black

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 3-pole, turned contacts with screw connection, snapable in housing, with integrated locking, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 2.5 mm²

Housing material: Polyamide

Connections per pole: 1

Marking: N, PE, L

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 S B1 RV SW

Part No.: 92.031.8758.1

      - Female connector with spring clamp connection

gesis® Snap-In, female, 3-pole, for panel mounting, 250 V, 20 A, white

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 3-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: N, PE, L

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F B2 RV WS

Part No.: 92.031.9658.0

gesis® Snap-In, female, 3-pole, for panel mounting, 250 V, 20 A, white

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 3-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 1.5 - 2.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: N, PE, L

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F B2 REV WS

Part No.: 92.031.7658.0

gesis® Snap-In, female, 3-pole, for panel mounting, 250 V, 20 A, black

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 3-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: N, PE, L

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F B2 RV SW

Part No.: 92.031.9658.1

gesis® Snap-In, female, 3-pole, for panel mounting, 250 V, 20 A, black

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 3-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 1.5 - 2.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: N, PE, L

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F B2 REV SW

Part No.: 92.031.7658.1

gesis® Snap-In, female, 3-pole, for panel mounting, 250 V, 20 A, pebble gray

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 3-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: N, PE, L

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F B2 RV GR01

Part No.: 92.031.9758.0

gesis® Snap-In, female, 3-pole, for panel mounting, 250 V, 20 A, light red

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 3-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 3, color coding light red.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: N, PE, L

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F B2 RV RT03

Part No.: 92.031.8558.0

gesis® Snap-In, female, 3-pole, for panel mounting, 250 V, 20 A, brown

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 3-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 4, color coding brown.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: 3, 2, 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F B2 RV BR05

Part No.: 92.031.8458.0

      - Male with screw connection

gesis® Snap-In, male, 3-pole, for panel mounting, 250 V, 20 A, white

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 3-pole, turned contacts with screw connection, snapable in housing, with integrated locking, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 2.5 mm²

Housing material: Polyamide

Connections per pole: 1

Marking: L, PE, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 S S1 RV WS

Part No.: 92.032.8758.0

gesis® Snap-In, male, 3-pole, for panel mounting, 250 V, 20 A, black

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 3-pole, turned contacts with screw connection, snapable in housing, with integrated locking, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 2.5 mm²

Housing material: Polyamide

Connections per pole: 1

Marking: L, PE, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 S S1 RV SW

Part No.: 92.032.8758.1

      - Male with spring clamp connection

gesis® Snap-In, male, 3-pole, for panel mounting, 250 V, 20 A, white

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 3-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: L, PE, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F S2 RV WS

Part No.: 92.032.9658.0

gesis® Snap-In, male, 3-pole, for panel mounting, 250 V, 20 A, white

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 3-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 1.5 - 2.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: L, PE, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F S2 REV WS

Part No.: 92.032.7658.0

gesis® Snap-In, male, 3-pole, for panel mounting, 250 V, 20 A, black

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 3-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: L, PE, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F S2 RV SW

Part No.: 92.032.9658.1

gesis® Snap-In, male, 3-pole, for panel mounting, 250 V, 20 A, black

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 3-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 1.5 - 2.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: L, PE, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F S2 REV SW

Part No.: 92.032.7658.1

gesis® Snap-In, male, 3-pole, for panel mounting, 250 V, 20 A, pebble gray

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 3-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: L, PE, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F S2 RV GR01

Part No.: 92.032.9758.0

gesis® Snap-In, male, 3-pole, for panel mounting, 250 V, 20 A, light red

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 3-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 3, color coding light red.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: L, PE, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F S2 RV RT03

Part No.: 92.032.8558.0

gesis® Snap-In, male, 3-pole, for panel mounting, 250 V, 20 A, brown

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 3-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 4, color coding brown.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: 1, 2, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 F S2 RV BR05

Part No.: 92.032.8458.0

    - Luminaire connection

gesis® pluggable luminaire connection adapter, 3-pole, 250 V, 20 A, white

System gesis® CON - gesis TOP adapter for direct, pluggable connection of corresponding assembled luminaires according to IEC 61535 for permanent connection, 3-pole, turned contacts, with integrated locking, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: IP20

Housing material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 V N 2P1 V WS

Part No.: 92.030.2153.0

gesis® pluggable luminaire connection adapter, 3-pole, 250 V, 20 A, white

System gesis® CON - gesis LINECT adapter for direct, pluggable connection of corresponding assembled luminaires according to IEC 61535 for permanent connection, 3-pole, turned contacts, with integrated locking, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: IP20

Housing material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 V N 2P1 N V WS

Part No.: 92.030.2253.0

    - Distribution blocks

gesis® distribution block, 3-pole, 1 input, 2 outputs, 250 V, 20 A, white

System gesis® CON - Distribution block for installation connectors according to IEC 61535, T-shaped 05.564.5353.2, 3-pole, turned contacts, 1 input, 2 outputs, with integrated locking, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 V 2P1 T V WS

Part No.: 92.030.1053.0

gesis® distribution block, 3-pole, 1 input, 2 outputs, 250 V, 20 A, black

System gesis® CON - Distribution block for installation connectors according to IEC 61535, T-shaped distribution block, 3-pole, turned contacts, 1 input, 2 outputs, with integrated locking, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 V 2P1 T V SW

Part No.: 92.030.1053.1

gesis® distribution block, 3-pole, 1 input, 2 outputs, 250 V, 20 A, white

System gesis® CON - Distribution block for installation connectors according to IEC 61535, T-shaped distribution block (flat version), 3-pole, turned contacts, 1 input (right), 2 outputs, with integrated locking, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 V 2P1 T SV WS

Part No.: 92.030.1253.0

gesis® distribution block, 3-pole, 1 input, 2 outputs, 250 V, 20 A, black

System gesis® CON - Distribution block for installation connectors according to IEC 61535, T-shaped distribution block (flat version), 3-pole, turned contacts, 1 input (right), 2 outputs, with integrated locking, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 V 2P1 T SV SW

Part No.: 92.030.1253.1

gesis® distribution block, 3-pole, 1 input, 2 outputs, 250 V, 20 A, black

System gesis® CON - Distribution block for installation connectors according to IEC 61535, T-shaped distribution block (flat version), 3-pole, turned contacts, 1 input (left), 2 outputs, with integrated locking, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 V 2P1 T IV SW

Part No.: 92.030.1353.0

gesis® distribution block, 3-pole, 1 input, 2 outputs, 250 V, 20 A, white

System gesis® CON - Distribution block for installation connectors according to IEC 61535, h-shaped distribution block, 3-pole, turned contacts, 1 input, 2 outputs, with integrated locking, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 V 2P1 h V WS

Part No.: 92.030.6053.0

gesis® distribution block, 3-pole, 1 input, 2 outputs, 250 V, 20 A, black

System gesis® CON - Distribution block for installation connectors according to IEC 61535, h-shaped distribution block, 3-pole, turned contacts, 1 input, 2 outputs, with integrated locking, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 V 2P1 h V SW

Part No.: 92.030.6053.1

gesis® distribution block with mounting option, 3-pole, 1 input, 3 outputs, 250 V, 20 A, white

System gesis® CON - Distribution block for installation connectors according to IEC 61535, H-shaped distribution block, 3-pole, turned contacts, 1 input, 3 outputs, with mounting option, with integrated locking, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 V 3P1 F V WS

Part No.: 92.030.4853.0

gesis® distribution block with mounting option, 3-pole, 1 input, 3 outputs, 250 V, 20 A, black

System gesis® CON - Distribution block for installation connectors according to IEC 61535, H-shaped distribution block, 3-pole, turned contacts, 1 input, 3 outputs, with mounting option, with integrated locking, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 V 3P1 F V SW

Part No.: 92.030.4853.1

gesis® distribution block, 3-pole, 1 input, 3 outputs, 250 V, 20 A, white

System gesis® CON - Distribution block for installation connectors according to IEC 61535, H-shaped distribution block, 3-pole, turned contacts, 1 input, 3 outputs, without mounting option, with integrated locking, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 V 3P1 F V WS

Part No.: 92.030.4953.0

gesis® distribution block, 3-pole, 1 input, 3 outputs, 250 V, 20 A, black

System gesis® CON - Distribution block for installation connectors according to IEC 61535, H-shaped distribution block, 3-pole, turned contacts, 1 input, 3 outputs, without mounting option, with integrated locking, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 V 3P1 F V SW

Part No.: 92.030.4953.1

gesis® distribution block with mounting option, 3-pole, 1 input, 5 outputs, 250 V, 20 A, white

System gesis® CON - Distribution block for installation connectors according to IEC 61535, distribution block, 3-pole, turned contacts, 1 input, 5 outputs, with mounting option, with integrated locking, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 5

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 V 5P1 F V WS

Part No.: 92.030.5353.0

gesis® distribution block with mounting option, 3-pole, 1 input, 5 outputs, 250 V, 20 A, black

System gesis® CON - Distribution block for installation connectors according to IEC 61535, H-shaped distribution block, 3-pole, turned contacts, 1 input, 5 outputs, with mounting option, with integrated locking, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 5

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 V 5P1 F V SW

Part No.: 92.030.5353.1

gesis® distribution block, 3-pole, 1 input, 5 outputs, 250 V, 20 A, white

System gesis® CON - Distribution block for installation connectors according to IEC 61535, H-shaped distribution block, 3-pole, turned contacts, 1 input, 5 outputs, without mounting option, with integrated locking, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 5

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 V 5P1 F V WS

Part No.: 92.030.5253.0

gesis® distribution block, 3-pole, 1 input, 5 outputs, 250 V, 20 A, black

System gesis® CON - Distribution block for installation connectors according to IEC 61535, H-shaped distribution block, 3-pole, turned contacts, 1 input, 5 outputs, without mounting option, with integrated locking, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 5

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 V 5P1 F V SW

Part No.: 92.030.5253.1

gesis® IDC adapter, 3-pole, 1 input, 2 outputs, 250 V, 20 A, white

System gesis® CON - GST18i3 female cable output 3-pole with IDC connection, useable for solid and stranded wires in existing installations 1.5 - 2.5 mm², 250 V, 20 A, mechanical coding code 1, color coding white, housing color white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Terminal cross-section, solid: 1.5 - 2.5 mm²

Terminal cross-section, 7-stranded: 1.5 mm²

Type of protection: up to IP40

Housing material: Polyamide

Input/forwarding: standard cable e.g. NYM

Output: 3-pole female GST18i3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 B2HZR1 M WS

Part No.: 92.731.6053.0

    - Accessories

gesis® intermediate coupling, 3-pole, 1 input, 1 output, 250 V, 20 A, white

System gesis® CON - Intermediate coupling for installation connectors according to IEC 61535, for direct forwarding e.g. between two luminaires, 3-pole, turned contacts, 1 input, 1 output, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Output: 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 Z 1P1 WS

Part No.: 92.030.5958.0

gesis® intermediate coupling, 3-pole, 1 input, 1 output, 250 V, 20 A, black

System gesis® CON - Intermediate coupling for installation connectors according to IEC 61535, for direct forwarding e.g. between two luminaires, 3-pole, turned contacts, 1 input, 1 output, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Output: 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 Z 1P1 SW

Part No.: 92.030.5958.1

gesis® intermediate coupling, 3-pole, 1 input, 1 output, 250 V, 20 A, black

System gesis® CON - Intermediate coupling for installation connectors according to IEC 61535, 3-pole, turned contacts, 1 input, 1 output, with locking option, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Output: 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 Z 1P1 SW

Part No.: 99.452.9910.0

gesis® U-piece, 3-pole, 1 input, 1 output, 250 V, 20 A, white

System gesis® CON - U-piece for installation connectors according to IEC 61535, 3-pole, turned contacts, 1 input, 1 output, with locking option, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Output: 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 Z 1P1 WS

Part No.: 92.030.5853.0

gesis® locking GST18, 3-, 4- or 5-pole, white

System gesis® CON - locking for solder parts GST18, 3-, 4- or 5-pole, white.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18 WS

Part No.: 05.590.4556.0

gesis® locking GST18, 3-, 4- or 5-pole, black

System gesis® CON - locking for solder parts GST18, 3-, 4- or 5-pole, black.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18 SW

Part No.: 05.590.4556.1

gesis® locking GST18, 3-, 4- or 5-pole, white

System gesis® CON - locking for floating cable connections GST18, 3-, 4- or 5-pole, white.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18 WS

Part No.: 05.587.3156.0

gesis® locking GST18, 3-, 4- or 5-pole, black

System gesis® CON - locking for floating cable connections GST18, 3-, 4- or 5-pole, black.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18 SW

Part No.: 05.587.3156.1

gesis® cover for female, 3-pole, white

System gesis® CON - Cover for female parts GST18i3, white.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 VS B WS

Part No.: 05.561.3653.0

gesis® cover for female, 3-pole, black

System gesis® CON - Cover for female parts GST18i3, black.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 VS B SW

Part No.: 05.561.3653.1

gesis® cover for male, 3-pole, white

System gesis® CON - Cover for male parts GST18i3, white.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 VS S WS

Part No.: 05.594.4250.0

gesis® cover for male, 3-pole, black

System gesis® CON - Cover for male parts GST18i3, black.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i3 VS S SW

Part No.: 05.594.4253.0

gesis® cover for female, 1-pole, white

System gesis® CON - Cover for female parts GST18, 3-, 4-, 5- or 6-pole, in a 12-pole pack, white.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18 WS

Part No.: 05.564.5353.2

gesis® cover for female, 1-pole, black

System gesis® CON - Cover for female parts GST18, 3-, 4-, 5- or 6-pole, in a 12-pole pack, black.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18 SW

Part No.: 05.564.5353.0

    - Cables

      - Cable assemblies, 1.5 mm², containing halogen

gesis® extension cable, female, male, 3x 1.5 mm², 250 V, 16 A, length: x m, white

System gesis® CON - assembled H05VV-F extension cable with protective ground, 3-pole, 1.5 mm², male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 BS 15 x0WS

Part No.: 92.232.x000.2 (x corresponds to length in m)

gesis® extension cable, female, male, 3x 1.5 mm², 250 V, 16 A, length: x m, black

System gesis® CON - assembled H05VV-F extension cable with protective ground, 3-pole, 1.5 mm², male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 BS 15 x0SW

Part No.: 92.232.x000.1 (x corresponds to length in m)

gesis® extension cable, female, male, 3x 1.5 mm², 250 V, 16 A, length: x m, pebble gray

System gesis® CON - assembled H05VV-F extension cable with protective ground, 3-pole, 1.5 mm², male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 BS 15 x0GR01

Part No.: 92.232.x000.3 (x corresponds to length in m)

gesis® extension cable, female, male, 3x 1.5 mm², 250 V, 16 A, length: x m, light red

System gesis® CON - assembled H05VV-F extension cable with protective ground, 3-pole, 1.5 mm², male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 3, color coding light red.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 BS 15 x0RT03

Part No.: 92.232.x000.5 (x corresponds to length in m)

gesis® connection cable, female, free end, 3x 1.5 mm², 250 V, 16 A, length: x m, white

System gesis® CON - assembled H05VV-F connection cable with protective ground, 3-pole, 1.5 mm², female GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 B- 15 x0WS

Part No.: 92.232.x003.2 (x corresponds to length in m)

gesis® connection cable, female, free end, 3x 1.5 mm², 250 V, 16 A, length: x m, black

System gesis® CON - assembled H05VV-F connection cable with protective ground, 3-pole, 1.5 mm², female GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 B- 15 x0SW

Part No.: 92.232.x003.1 (x corresponds to length in m)

gesis® connection cable, female, free end, 3x 1.5 mm², 250 V, 16 A, length: x m, pebble gray

System gesis® CON - assembled H05VV-F connection cable with protective ground, 3-pole, 1.5 mm², female GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 B- 15 x0GR01

Part No.: 92.232.x003.3 (x corresponds to length in m)

gesis® connection cable, female, free end, 3x 1.5 mm², 250 V, 16 A, length: x m, light red

System gesis® CON - assembled H05VV-F connection cable with protective ground, 3-pole, 1.5 mm², female GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 3, color coding light red.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 B- 15 x0RT03

Part No.: 92.232.x003.5 (x corresponds to length in m)

gesis® connection cable, male, free end, 3x 1.5 mm², 250 V, 16 A, length: x m, white

System gesis® CON - assembled H05VV-F connection cable with protective ground, 3-pole, 1.5 mm², male GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 -S 15 x0WS

Part No.: 92.232.x004.2 (x corresponds to length in m)

gesis® connection cable, male, free end, 3x 1.5 mm², 250 V, 16 A, length: x m, black

System gesis® CON - assembled H05VV-F connection cable with protective ground, 3-pole, 1.5 mm², male GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 -S 15 x0SW

Part No.: 92.232.x004.1 (x corresponds to length in m)

gesis® connection cable, male, free end, 3x 1.5 mm², 250 V, 16 A, length: x m, pebble gray

System gesis® CON - assembled H05VV-F connection cable with protective ground, 3-pole, 1.5 mm², male GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 -S 15 x0GR01

Part No.: 92.232.x004.3 (x corresponds to length in m)

gesis® connection cable, male, free end, 3x 1.5 mm², 250 V, 16 A, length: x m, light red

System gesis® CON - assembled H05VV-F connection cable with protective ground, 3-pole, 1.5 mm², male GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 3, color coding light red.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 -S 15 x0RT03

Part No.: 92.232.x004.5 (x corresponds to length in m)

      - Cable assemblies, 1.5 mm², halogen-free

gesis® extension cable, female, male, 3x 1.5 mm², 250 V, 16 A, length: x m, white

System gesis® CON - assembled S05Z1Z1-F extension cable with protective ground, 3-pole, 1.5 mm², male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: white

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 BS 15H x0WS

Part No.: 92.232.x060.2 (x corresponds to length in m)

gesis® extension cable, female, male, 3x 1.5 mm², 250 V, 16 A, length: x m, black

System gesis® CON - assembled S05Z1Z1-F extension cable with protective ground, 3-pole, 1.5 mm², male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 BS 15H x0SW

Part No.: 92.232.x060.1 (x corresponds to length in m)

gesis® extension cable, female, male, 3x 1.5 mm², 250 V, 16 A, length: x m, pebble gray

System gesis® CON - assembled S05Z1Z1-F extension cable with protective ground, 3-pole, 1.5 mm², male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 BS 15H x0GR01

Part No.: 92.232.x060.3 (x corresponds to length in m)

gesis® extension cable, female, male, 3x 1.5 mm², 250 V, 16 A, length: x m, light red

System gesis® CON - assembled S05Z1Z1-F extension cable with protective ground, 3-pole, 1.5 mm², male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 3, color coding light red.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 BS 15H x0RT03

Part No.: 92.232.x060.5 (x corresponds to length in m)

gesis® connection cable, female, free end, 3x 1.5 mm², 250 V, 16 A, length: x m, white

System gesis® CON - assembled S05Z1Z1-F connection cable with protective ground, 3-pole, 1.5 mm², female GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: white

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 B- 15H x0WS

Part No.: 92.232.x063.2 (x corresponds to length in m)

gesis® connection cable, female, free end, 3x 1.5 mm², 250 V, 16 A, length: x m, black

System gesis® CON - assembled S05Z1Z1-F connection cable with protective ground, 3-pole, 1.5 mm², female GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 B- 15H x0SW

Part No.: 92.232.x063.1 (x corresponds to length in m)

gesis® connection cable, female, free end, 3x 1.5 mm², 250 V, 16 A, length: x m, pebble gray

System gesis® CON - assembled S05Z1Z1-F connection cable with protective ground, 3-pole, 1.5 mm², female GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 B- 15H x0GR01

Part No.: 92.232.x063.3 (x corresponds to length in m)

gesis® connection cable, female, free end, 3x 1.5 mm², 250 V, 16 A, length: x m, light red

System gesis® CON - assembled S05Z1Z1-F connection cable with protective ground, 3-pole, 1.5 mm², female GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 3, color coding light red.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 B- 15H x0RT03

Part No.: 92.232.x063.5 (x corresponds to length in m)

gesis® connection cable, male, free end, 3x 1.5 mm², 250 V, 16 A, length: x m, white

System gesis® CON - assembled S05Z1Z1-F connection cable with protective ground, 3-pole, 1.5 mm², male GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: white

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 -S 15H x0WS

Part No.: 92.232.x064.2 (x corresponds to length in m)

gesis® connection cable, male, free end, 3x 1.5 mm², 250 V, 16 A, length: x m, black

System gesis® CON - assembled S05Z1Z1-F connection cable with protective ground, 3-pole, 1.5 mm², male GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 -S 15H x0SW

Part No.: 92.232.x064.1 (x corresponds to length in m)

gesis® connection cable, male, free end, 3x 1.5 mm², 250 V, 16 A, length: x m, pebble gray

System gesis® CON - assembled S05Z1Z1-F connection cable with protective ground, 3-pole, 1.5 mm², male GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 -S 15H x0GR01

Part No.: 92.232.x064.3 (x corresponds to length in m)

gesis® connection cable, male, free end, 3x 1.5 mm², 250 V, 16 A, length: x m, light red

System gesis® CON - assembled S05Z1Z1-F connection cable with protective ground, 3-pole, 1.5 mm², male GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 3, color coding light red.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 -S 15H x0RT03

Part No.: 92.232.x064.5 (x corresponds to length in m)

      - Cable assemblies, 1.5 mm², containing halogen, 90 °C

gesis® extension cable, female, male, 3x 1.5 mm², 250 V, 16 A, length: x m, white

System gesis® CON - assembled H05V2V2-F extension cable with protective ground, 3-pole, 1.5 mm², male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05V2V2-F

Continuous temperature: 90 °C

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 BS 15W x0WS

Part No.: 92.232.x010.2 (x corresponds to length in m)

gesis® extension cable, female, male, 3x 1.5 mm², 250 V, 16 A, length: x m, black

System gesis® CON - assembled H05V2V2-F extension cable with protective ground, 3-pole, 1.5 mm², male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05V2V2-F

Continuous temperature: 90 °C

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 BS 15W x0SW

Part No.: 92.232.x010.1 (x corresponds to length in m)

gesis® connection cable, female, free end, 3x 1.5 mm², 250 V, 16 A, length: x m, white

System gesis® CON - assembled H05V2V2-F connection cable with protective ground, 3-pole, 1.5 mm², female GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05V2V2-F

Continuous temperature: 90 °C

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 B- 15W x0WS

Part No.: 92.232.x013.2 (x corresponds to length in m)

gesis® connection cable, female, free end, 3x 1.5 mm², 250 V, 16 A, length: x m, black

System gesis® CON - assembled H05V2V2-F connection cable with protective ground, 3-pole, 1.5 mm², female GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05V2V2-F

Continuous temperature: 90 °C

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 B- 15W x0SW

Part No.: 92.232.x013.1 (x corresponds to length in m)

gesis® connection cable, male, free end, 3x 1.5 mm², 250 V, 16 A, length: x m, white

System gesis® CON - assembled H05V2V2-F connection cable with protective ground, 3-pole, 1.5 mm², male GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05V2V2-F

Continuous temperature: 90 °C

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 -S 15W x0WS

Part No.: 92.232.x014.2 (x corresponds to length in m)

gesis® connection cable, male, free end, 3x 1.5 mm², 250 V, 16 A, length: x m, black

System gesis® CON - assembled H05V2V2-F connection cable with protective ground, 3-pole, 1.5 mm², male GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05V2V2-F

Continuous temperature: 90 °C

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 -S 15W x0SW

Part No.: 92.232.x014.1 (x corresponds to length in m)

      - Cable assemblies, 2.5 mm², containing halogen

gesis® extension cable, female, male, 3x 2.5 mm², 250 V, 20 A, length: x m, white

System gesis® CON - assembled H05VV-F extension cable with protective ground, 3-pole, 2.5 mm², male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 BS 25 x0WS

Part No.: 92.238.x000.2 (x corresponds to length in m)

gesis® extension cable, female, male, 3x 2.5 mm², 250 V, 20 A, length: x m, black

System gesis® CON - assembled H05VV-F extension cable with protective ground, 3-pole, 2.5 mm², male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 BS 25 x0SW

Part No.: 92.238.x000.1 (x corresponds to length in m)

gesis® extension cable, female, male, 3x 2.5 mm², 250 V, 20 A, length: x m, pebble gray

System gesis® CON - assembled H05VV-F extension cable with protective ground, 3-pole, 2.5 mm², male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 BS 25 x0GR01

Part No.: 92.238.x000.3 (x corresponds to length in m)

gesis® extension cable, female, male, 3x 2.5 mm², 250 V, 20 A, length: x m, light red

System gesis® CON - assembled H05VV-F extension cable with protective ground, 3-pole, 2.5 mm², male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 3, color coding light red.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 BS 25 x0RT03

Part No.: 92.238.x000.5 (x corresponds to length in m)

gesis® connection cable, female, free end, 3x 2.5 mm², 250 V, 20 A, length: x m, white

System gesis® CON - assembled H05VV-F connection cable with protective ground, 3-pole, 2.5 mm², female GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 B- 25 x0WS

Part No.: 92.238.x003.2 (x corresponds to length in m)

gesis® connection cable, female, free end, 3x 2.5 mm², 250 V, 20 A, length: x m, black

System gesis® CON - assembled H05VV-F connection cable with protective ground, 3-pole, 2.5 mm², female GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 B- 25 x0SW

Part No.: 92.238.x003.1 (x corresponds to length in m)

gesis® connection cable, female, free end, 3x 2.5 mm², 250 V, 20 A, length: x m, pebble gray

System gesis® CON - assembled H05VV-F connection cable with protective ground, 3-pole, 2.5 mm², female GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 B- 25 x0GR01

Part No.: 92.238.x003.3 (x corresponds to length in m)

gesis® connection cable, female, free end, 3x 2.5 mm², 250 V, 20 A, length: x m, light red

System gesis® CON - assembled H05VV-F connection cable with protective ground, 3-pole, 2.5 mm², female GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 3, color coding light red.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 B- 25 x0RT03

Part No.: 92.238.x003.5 (x corresponds to length in m)

gesis® connection cable, male, free end, 3x 2.5 mm², 250 V, 20 A, length: x m, white

System gesis® CON - assembled H05VV-F connection cable with protective ground, 3-pole, 2.5 mm², male GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 -S 25 x0WS

Part No.: 92.238.x004.2 (x corresponds to length in m)

gesis® connection cable, male, free end, 3x 2.5 mm², 250 V, 20 A, length: x m, black

System gesis® CON - assembled H05VV-F connection cable with protective ground, 3-pole, 2.5 mm², male GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 -S 25 x0SW

Part No.: 92.238.x004.1 (x corresponds to length in m)

gesis® connection cable, male, free end, 3x 2.5 mm², 250 V, 20 A, length: x m, pebble gray

System gesis® CON - assembled H05VV-F connection cable with protective ground, 3-pole, 2.5 mm², male GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 -S 25 x0GR01

Part No.: 92.238.x004.3 (x corresponds to length in m)

gesis® connection cable, male, free end, 3x 2.5 mm², 250 V, 20 A, length: x m, light red

System gesis® CON - assembled H05VV-F connection cable with protective ground, 3-pole, 2.5 mm², male GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 3, color coding light red.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 -S 25 x0RT03

Part No.: 92.238.x004.5 (x corresponds to length in m)

      - Cable assemblies, 2.5 mm², halogen-free

gesis® extension cable, female, male, 3x 2.5 mm², 250 V, 20 A, length: x m, white

System gesis® CON - assembled S05Z1Z1-F extension cable with protective ground, 3-pole, 2.5 mm², male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: white

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 BS 25H x0WS

Part No.: 92.238.x060.2 (x corresponds to length in m)

gesis® extension cable, female, male, 3x 2.5 mm², 250 V, 20 A, length: x m, black

System gesis® CON - assembled S05Z1Z1-F extension cable with protective ground, 3-pole, 2.5 mm², male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 BS 25H x0SW

Part No.: 92.238.x060.1 (x corresponds to length in m)

gesis® extension cable, female, male, 3x 2.5 mm², 250 V, 20 A, length: x m, pebble gray

System gesis® CON - assembled S05Z1Z1-F extension cable with protective ground, 3-pole, 2.5 mm², male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 BS 25H x0GR01

Part No.: 92.238.x060.3 (x corresponds to length in m)

gesis® extension cable, female, male, 3x 2.5 mm², 250 V, 20 A, length: x m, light red

System gesis® CON - assembled S05Z1Z1-F extension cable with protective ground, 3-pole, 2.5 mm², male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 3, color coding light red.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 BS 25H x0RT03

Part No.: 92.238.x060.5 (x corresponds to length in m)

gesis® connection cable, female, free end, 3x 2.5 mm², 250 V, 20 A, length: x m, white

System gesis® CON - assembled S05Z1Z1-F connection cable with protective ground, 3-pole, 2.5 mm², female GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: white

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 B- 25H x0WS

Part No.: 92.238.x063.2 (x corresponds to length in m)

gesis® connection cable, female, free end, 3x 2.5 mm², 250 V, 20 A, length: x m, black

System gesis® CON - assembled S05Z1Z1-F connection cable with protective ground, 3-pole, 2.5 mm², female GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 B- 25H x0SW

Part No.: 92.238.x063.1 (x corresponds to length in m)

gesis® connection cable, female, free end, 3x 2.5 mm², 250 V, 20 A, length: x m, pebble gray

System gesis® CON - assembled S05Z1Z1-F connection cable with protective ground, 3-pole, 2.5 mm², female GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 B- 25H x0GR01

Part No.: 92.238.x063.3 (x corresponds to length in m)

gesis® connection cable, female, free end, 3x 2.5 mm², 250 V, 20 A, length: x m, light red

System gesis® CON - assembled S05Z1Z1-F connection cable with protective ground, 3-pole, 2.5 mm², female GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 3, color coding light red.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 B- 25H x0RT03

Part No.: 92.238.x063.5 (x corresponds to length in m)

gesis® connection cable, male, free end, 3x 2.5 mm², 250 V, 20 A, length: x m, white

System gesis® CON - assembled S05Z1Z1-F connection cable with protective ground, 3-pole, 2.5 mm², male GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: white

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 -S 25H x0WS

Part No.: 92.238.x064.2 (x corresponds to length in m)

gesis® connection cable, male, free end, 3x 2.5 mm², 250 V, 20 A, length: x m, black

System gesis® CON - assembled S05Z1Z1-F connection cable with protective ground, 3-pole, 2.5 mm², male GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 -S 25H x0SW

Part No.: 92.238.x064.1 (x corresponds to length in m)

gesis® connection cable, male, free end, 3x 2.5 mm², 250 V, 20 A, length: x m, pebble gray

System gesis® CON - assembled S05Z1Z1-F connection cable with protective ground, 3-pole, 2.5 mm², male GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 -S 25H x0GR01

Part No.: 92.238.x064.3 (x corresponds to length in m)

gesis® connection cable, male, free end, 3x 2.5 mm², 250 V, 20 A, length: x m, light red

System gesis® CON - assembled S05Z1Z1-F connection cable with protective ground, 3-pole, 2.5 mm², male GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 3, color coding light red.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 -S 25H x0RT03

Part No.: 92.238.x064.5 (x corresponds to length in m)

      - Cable assemblies, 2.5 mm², containing halogen, 90 °C

gesis® extension cable, female, male, 3x 2.5 mm², 250 V, 20 A, length: x m, white

System gesis® CON - assembled H05V2V2-F extension cable with protective ground, 3-pole, 2.5 mm², male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05V2V2-F

Continuous temperature: 90 °C

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 BS 25W x0WS

Part No.: 92.238.x010.2 (x corresponds to length in m)

gesis® extension cable, female, male, 3x 2.5 mm², 250 V, 20 A, length: x m, black

System gesis® CON - assembled H05V2V2-F extension cable with protective ground, 3-pole, 2.5 mm², male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05V2V2-F

Continuous temperature: 90 °C

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 BS 25W x0SW

Part No.: 92.238.x010.1 (x corresponds to length in m)

gesis® connection cable, female, free end, 3x 2.5 mm², 250 V, 20 A, length: x m, white

System gesis® CON - assembled H05V2V2-F connection cable with protective ground, 3-pole, 2.5 mm², female GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05V2V2-F

Continuous temperature: 90 °C

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 B- 25W x0WS

Part No.: 92.238.x013.2 (x corresponds to length in m)

gesis® connection cable, female, free end, 3x 2.5 mm², 250 V, 20 A, length: x m, black

System gesis® CON - assembled H05V2V2-F connection cable with protective ground, 3-pole, 2.5 mm², female GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05V2V2-F

Continuous temperature: 90 °C

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 B- 25W x0SW

Part No.: 92.238.x013.1 (x corresponds to length in m)

gesis® connection cable, male, free end, 3x 2.5 mm², 250 V, 20 A, length: x m, white

System gesis® CON - assembled H05V2V2-F connection cable with protective ground, 3-pole, 2.5 mm², male GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05V2V2-F

Continuous temperature: 90 °C

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 -S 25W x0WS

Part No.: 92.238.x014.2 (x corresponds to length in m)

gesis® connection cable, male, free end, 3x 2.5 mm², 250 V, 20 A, length: x m, black

System gesis® CON - assembled H05V2V2-F connection cable with protective ground, 3-pole, 2.5 mm², male GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05V2V2-F

Continuous temperature: 90 °C

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 -S 25W x0SW

Part No.: 92.238.x014.1 (x corresponds to length in m)

    - Outlets

gesis® SCHUKO-socket outlet with GST18 forwarding, 2-fold, white

System gesis® CON - SCHUKO-socket outlet, 2-fold, with integrated locking, infeed/forwarding via male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 16 A

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Color: white

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: WSDL 1-2 WS

Part No.: 99.612.0028.0

gesis® SCHUKO-socket outlet with GST18 forwarding, 2-fold, black

System gesis® CON - SCHUKO-socket outlet, 2-fold, with integrated locking, infeed/forwarding via male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 16 A

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Color: black

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: WSDL 1-2 SW

Part No.: 99.600.0028.0

gesis® SCHUKO-socket outlet with GST18 forwarding, 3-fold, white

System gesis® CON - SCHUKO-socket outlet, 3-fold, with integrated locking, infeed/forwarding via male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 16 A

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Color: white

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: WSDL 1-3 WS

Part No.: 99.602.0028.0

gesis® SCHUKO-socket outlet with GST18 forwarding, 3-fold, black

System gesis® CON - SCHUKO-socket outlet, 3-fold, with integrated locking, infeed/forwarding via male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 16 A

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Color: black

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: WSDL 1-3 SW

Part No.: 99.601.0028.0

gesis® SCHUKO-socket outlet with GST18 forwarding, 3-fold, contact protection, white

System gesis® CON - SCHUKO-socket outlet, 3-fold, with integrated locking, with contact protection, infeed/forwarding via male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 16 A

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Color: white

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: WSDL 1-3 WS

Part No.: 99.653.0028.0

gesis® SCHUKO-socket outlet with GST18 forwarding, 3-fold, contact protection, black

System gesis® CON - SCHUKO-socket outlet, 3-fold, with integrated locking, with contact protection, infeed/forwarding via male GST18i3, female GST18i3, according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 16 A

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Color: black

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: WSDL 1-3 SW

Part No.: 99.652.0028.0

    - Assembled Cables

gesis® cable, female, SCHUKO male plug, 3x 1.5 mm², 250 V, 16 A, length: x m, white

System gesis® CON - assembled H05VV-F cable with protective ground, 3-pole, 1.5 mm², female GST18i3 with SCHUKO male plug according to DIN 49441, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 BD 15 x0WS

Part No.: 92.232.x007.2 (x corresponds to length in m)

gesis® cable, female, SCHUKO male plug, 3x 1.5 mm², 250 V, 16 A, length: x m, black

System gesis® CON - assembled H05VV-F cable with protective ground, 3-pole, 1.5 mm², female GST18i3 with SCHUKO male plug according to DIN 49441, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 BD 15 x0SW

Part No.: 92.232.x007.1 (x corresponds to length in m)

gesis® cable, male, low-heat device female plug, 3x 1.0 mm², 250 V, 10 A, length: 2 m, white

System gesis® CON - assembled H05VV-F cable with protective ground, 3-pole, 1.0 mm², male GST18i3 with low-heat device female plug, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: white

Halogen-free: no

Length: 2 m

Brand: Wieland Electric GmbH

Type: GST18i3 K1 KS 10 20WS

Part No.: 99.400.5623.1

  - 4-pole - GST18i4

    - Connectors

      - Female connector with screw connection

gesis® connector, female, 4-pole, 250/400 V, 20 A, white

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 4-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 7.1 - 12.0 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: 2, N, PE, 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 S B1 Z R1 WS

Part No.: 92.943.3053.0

gesis® connector, female, 4-pole, 250/400 V, 20 A, black

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 4-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 7.1 - 12.0 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: 2, N, PE, 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 S B1 Z R1 SW

Part No.: 92.943.3053.1

gesis® connector, female, 4-pole, 250/400 V, 20 A, pebble gray

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 4-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 7.1 - 12.0 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: 2, N, PE, 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 S B1 Z R1 GR01

Part No.: 92.943.3553.0

      - Female connector with spring clamp connection

gesis® connector, female, 4-pole, 250/400 V, 20 A, white

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 4-pole, turned contacts with spring clamp connection, compact housing with strain relief, field assembly, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 1.5 - 2.5 mm²

Terminal cross-section, stranded: 1.5 mm² with ferrule

Cable diameter: 9.5 - 11.5 mm (H05VV, NYM, H05RNF)

Housing material: Polyamide

Connections per pole: 2

Marking: 2, N, PE, 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 F B2 Z R1 WS

Part No.: 92.943.0053.0

gesis® connector, female, 4-pole, 250/400 V, 20 A, black

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 4-pole, turned contacts with spring clamp connection, compact housing with strain relief, field assembly, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 1.5 - 2.5 mm²

Terminal cross-section, stranded: 1.5 mm² with ferrule

Cable diameter: 9.5 - 11.5 mm (H05VV, NYM, H05RNF)

Housing material: Polyamide

Connections per pole: 2

Marking: 2, N, PE, 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 F B2 Z R1 SW

Part No.: 92.943.0053.1

gesis® connector, female, 4-pole, 250/400 V, 20 A, pebble gray

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 4-pole, turned contacts with spring clamp connection, compact housing with strain relief, field assembly, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 1.5 - 2.5 mm²

Terminal cross-section, stranded: 1.5 mm² with ferrule

Cable diameter: 9.5 - 11.5 mm (H05VV, NYM, H05RNF)

Housing material: Polyamide

Connections per pole: 2

Marking: 2, N, PE, 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 F B2 Z R1 GR01

Part No.: 92.943.0153.0

      - Male with screw connection

gesis® connector, male, 4-pole, 250/400 V, 20 A, white

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 4-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 7.1 - 12.0 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: 1, PE, N, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 S S1 Z R1 WS

Part No.: 92.944.3053.0

gesis® connector, male, 4-pole, 250/400 V, 20 A, black

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 4-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 7.1 - 12.0 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: 1, PE, N, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 S S1 Z R1 SW

Part No.: 92.944.3053.1

gesis® connector, male, 4-pole, 250/400 V, 20 A, pebble gray

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 4-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 7.1 - 12.0 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: 1, PE, N, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 S S1 Z R1 GR01

Part No.: 92.944.3553.0

      - Male with spring clamp connection

gesis® connector, male, 4-pole, 250/400 V, 20 A, white

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 4-pole, turned contacts with spring clamp connection, compact housing with strain relief, field assembly, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 1.5 - 2.5 mm²

Terminal cross-section, stranded: 1.5 mm² with ferrule

Cable diameter: 9.5 - 11.5 mm (H05VV, NYM, H05RNF)

Housing material: Polyamide

Connections per pole: 2

Marking: 1, PE, N, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 F S2 Z R1 WS

Part No.: 92.944.0053.0

gesis® connector, male, 4-pole, 250/400 V, 20 A, black

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 4-pole, turned contacts with spring clamp connection, compact housing with strain relief, field assembly, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 1.5 - 2.5 mm²

Terminal cross-section, stranded: 1.5 mm² with ferrule

Cable diameter: 9.5 - 11.5 mm (H05VV, NYM, H05RNF)

Housing material: Polyamide

Connections per pole: 2

Marking: 1, PE, N, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 F S2 Z R1 SW

Part No.: 92.944.0053.1

gesis® connector, male, 4-pole, 250/400 V, 20 A, pebble gray

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 4-pole, turned contacts with spring clamp connection, compact housing with strain relief, field assembly, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 1.5 - 2.5 mm²

Terminal cross-section, stranded: 1.5 mm² with ferrule

Cable diameter: 9.5 - 11.5 mm (H05VV, NYM, H05RNF)

Housing material: Polyamide

Connections per pole: 2

Marking: 1, PE, N, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 F S2 Z R1 GR01

Part No.: 92.944.0153.0

    - Snap-In

      - Female connector with spring clamp connection

gesis® Snap-In, female, 4-pole, for panel mounting, 250/400 V, 20 A, white

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 4-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: 2, N, PE, 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 F B2 RV WS

Part No.: 92.041.9658.0

gesis® Snap-In, female, 4-pole, for panel mounting, 250/400 V, 20 A, black

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 4-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: 2, N, PE, 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 F B2 RV SW

Part No.: 92.041.9658.1

gesis® Snap-In, female, 4-pole, for panel mounting, 250/400 V, 20 A, pebble gray

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 4-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: 2, N, PE, 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 F B2 RV GR01

Part No.: 92.041.9758.0

      - Male with spring clamp connection

gesis® Snap-In, male, 4-pole, for panel mounting, 250/400 V, 20 A, white

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 4-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: 1, PE, N, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 F S2 RV WS

Part No.: 92.042.9658.0

gesis® Snap-In, male, 4-pole, for panel mounting, 250/400 V, 20 A, black

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 4-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: 1, PE, N, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 F S2 RV SW

Part No.: 92.042.9658.1

    - Distribution blocks

gesis® distribution block, 4-pole, 1 input, 2 outputs, 250/400 V, 20 A, white

System gesis® CON - Distribution block for installation connectors according to IEC 61535, T-shaped distribution block, 4-pole, turned contacts, 1 input, 2 outputs, with integrated locking, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 V 2P1 T V WS

Part No.: 92.040.1053.0

gesis® distribution block, 4-pole, 1 input, 2 outputs, 250/400 V, 20 A, black

System gesis® CON - Distribution block for installation connectors according to IEC 61535, T-shaped distribution block, 4-pole, turned contacts, 1 input, 2 outputs, with integrated locking, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 V 2P1 T V SW

Part No.: 92.040.1053.1

gesis® distribution block, 4-pole, 1 input, 2 outputs, 250/400 V, 20 A, pebble gray

System gesis® CON - Distribution block for installation connectors according to IEC 61535, T-shaped distribution block, 4-pole, turned contacts, 1 input, 2 outputs, with integrated locking, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 V 2P1 T V GR01

Part No.: 92.040.1153.0

    - Accessories

gesis® intermediate coupling, 4-pole, 1 input, 1 output, 250/400 V, 20 A, black

System gesis® CON - Intermediate coupling for installation connectors according to IEC 61535, for direct forwarding e.g. between two luminaires, 4-pole, turned contacts, 1 input, 1 output, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Output: 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 Z 1P1 SW

Part No.: 99.453.9910.0

gesis® locking GST18, 3-, 4- or 5-pole, white

System gesis® CON - locking for solder parts GST18, 3-, 4- or 5-pole, white.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18 WS

Part No.: 05.590.4556.0

gesis® locking GST18, 3-, 4- or 5-pole, black

System gesis® CON - locking for solder parts GST18, 3-, 4- or 5-pole, black.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18 SW

Part No.: 05.590.4556.1

gesis® locking GST18, 3-, 4- or 5-pole, white

System gesis® CON - locking for floating cable connections GST18, 3-, 4- or 5-pole, white.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18 WS

Part No.: 05.587.3156.0

gesis® locking GST18, 3-, 4- or 5-pole, black

System gesis® CON - locking for floating cable connections GST18, 3-, 4- or 5-pole, black.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18 SW

Part No.: 05.587.3156.1

gesis® cover for female, 4-pole, black

System gesis® CON - Cover for female parts GST18i4, black.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 VS B SW

Part No.: 05.566.9153.0

gesis® cover for female, 1-pole, white

System gesis® CON - Cover for female parts GST18, 3-, 4-, 5- or 6-pole, in a 12-pole pack, white.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18 WS

Part No.: 05.564.5353.2

gesis® cover for female, 1-pole, black

System gesis® CON - Cover for female parts GST18, 3-, 4-, 5- or 6-pole, in a 12-pole pack, black.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18 SW

Part No.: 05.564.5353.0

    - Cables

      - Cable assemblies, 1.5 mm², containing halogen

gesis® extension cable, female, male, 4x 1.5 mm², 250/400 V, 16 A, length: x m, white

System gesis® CON - assembled H05VV-F extension cable with protective ground, 4-pole, 1.5 mm², male GST18i4, female GST18i4, according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 BS 15 x0WS

Part No.: 92.207.x000.2 (x corresponds to length in m)

gesis® extension cable, female, male, 4x 1.5 mm², 250/400 V, 16 A, length: x m, black

System gesis® CON - assembled H05VV-F extension cable with protective ground, 4-pole, 1.5 mm², male GST18i4, female GST18i4, according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 BS 15 x0SW

Part No.: 92.207.x000.1 (x corresponds to length in m)

gesis® extension cable, female, male, 4x 1.5 mm², 250/400 V, 16 A, length: x m, pebble gray

System gesis® CON - assembled H05VV-F extension cable with protective ground, 4-pole, 1.5 mm², male GST18i4, female GST18i4, according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 BS 15 x0GR01

Part No.: 92.207.x000.3 (x corresponds to length in m)

gesis® connection cable, female, free end, 4x 1.5 mm², 250/400 V, 16 A, length: x m, white

System gesis® CON - assembled H05VV-F connection cable with protective ground, 4-pole, 1.5 mm², female GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 B- 15 x0WS

Part No.: 92.207.x003.2 (x corresponds to length in m)

gesis® connection cable, female, free end, 4x 1.5 mm², 250/400 V, 16 A, length: x m, black

System gesis® CON - assembled H05VV-F connection cable with protective ground, 4-pole, 1.5 mm², female GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 B- 15 x0SW

Part No.: 92.207.x003.1 (x corresponds to length in m)

gesis® connection cable, female, free end, 4x 1.5 mm², 250/400 V, 16 A, length: x m, pebble gray

System gesis® CON - assembled H05VV-F connection cable with protective ground, 4-pole, 1.5 mm², female GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 B- 15 x0GR01

Part No.: 92.207.x003.3 (x corresponds to length in m)

gesis® connection cable, male, free end, 4x 1.5 mm², 250/400 V, 16 A, length: x m, white

System gesis® CON - assembled H05VV-F connection cable with protective ground, 4-pole, 1.5 mm², male GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 -S 15 x0WS

Part No.: 92.207.x004.2 (x corresponds to length in m)

gesis® connection cable, male, free end, 4x 1.5 mm², 250/400 V, 16 A, length: x m, black

System gesis® CON - assembled H05VV-F connection cable with protective ground, 4-pole, 1.5 mm², male GST18i3, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 -S 15 x0SW

Part No.: 92.207.x004.1 (x corresponds to length in m)

gesis® connection cable, male, free end, 4x 1.5 mm², 250/400 V, 16 A, length: x m, pebble gray

System gesis® CON - assembled H05VV-F connection cable with protective ground, 4-pole, 1.5 mm², male GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 -S 15 x0GR01

Part No.: 92.207.x004.3 (x corresponds to length in m)

      - Cable assemblies, 1.5 mm², halogen-free

gesis® extension cable, female, male, 4x 1.5 mm², 250/400 V, 16 A, length: x m, white

System gesis® CON - assembled H05Z1Z1-F extension cable with protective ground, 4-pole, 1.5 mm², male GST18i4, female GST18i4, according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: white

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 BS 15H x0WS

Part No.: 92.207.x060.1 (x corresponds to length in m)

gesis® extension cable, female, male, 4x 1.5 mm², 250/400 V, 16 A, length: x m, black

System gesis® CON - assembled H05Z1Z1-F extension cable with protective ground, 4-pole, 1.5 mm², male GST18i4, female GST18i4, according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 BS 15H x0SW

Part No.: 92.207.x060.2 (x corresponds to length in m)

gesis® extension cable, female, male, 4x 1.5 mm², 250/400 V, 16 A, length: x m, pebble gray

System gesis® CON - assembled H05Z1Z1-F extension cable with protective ground, 4-pole, 1.5 mm², male GST18i4, female GST18i4, according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 BS 15H x0GR01

Part No.: 92.207.x060.3 (x corresponds to length in m)

gesis® connection cable, female, free end, 4x 1.5 mm², 250/400 V, 16 A, length: x m, white

System gesis® CON - assembled H05Z1Z1-F connection cable with protective ground, 4-pole, 1.5 mm², female GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: white

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 B- 15H x0WS

Part No.: 92.207.x063.2 (x corresponds to length in m)

gesis® connection cable, female, free end, 4x 1.5 mm², 250/400 V, 16 A, length: x m, black

System gesis® CON - assembled H05Z1Z1-F connection cable with protective ground, 4-pole, 1.5 mm², female GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 B- 15H x0SW

Part No.: 92.207.x063.1 (x corresponds to length in m)

gesis® connection cable, female, free end, 4x 1.5 mm², 250/400 V, 16 A, length: x m, pebble gray

System gesis® CON - assembled H05Z1Z1-F connection cable with protective ground, 4-pole, 1.5 mm², female GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 B- 15H x0GR01

Part No.: 92.207.x063.3 (x corresponds to length in m)

gesis® connection cable, male, free end, 4x 1.5 mm², 250/400 V, 16 A, length: x m, white

System gesis® CON - assembled H05Z1Z1-F connection cable with protective ground, 4-pole, 1.5 mm², male GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: white

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 -S 15H x0WS

Part No.: 92.207.x064.2 (x corresponds to length in m)

gesis® connection cable, male, free end, 4x 1.5 mm², 250/400 V, 16 A, length: x m, black

System gesis® CON - assembled H05Z1Z1-F connection cable with protective ground, 4-pole, 1.5 mm², male GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 -S 15H x0SW

Part No.: 92.207.x064.1 (x corresponds to length in m)

gesis® connection cable, male, free end, 4x 1.5 mm², 250/400 V, 16 A, length: x m, pebble gray

System gesis® CON - assembled H05Z1Z1-F connection cable with protective ground, 4-pole, 1.5 mm², male GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 -S 15H x0GR01

Part No.: 92.207.x064.3 (x corresponds to length in m)

      - Cable assemblies, 2.5 mm², containing halogen

gesis® extension cable, female, male, 4x 2.5 mm², 250/400 V, 20 A, length: x m, white

System gesis® CON - assembled H05VV-F extension cable with protective ground, 4-pole, 2.5 mm², male GST18i4, female GST18i4, according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 BS 25 x0WS

Part No.: 92.208.x000.2 (x corresponds to length in m)

gesis® extension cable, female, male, 4x 2.5 mm², 250/400 V, 20 A, length: x m, black

System gesis® CON - assembled H05VV-F extension cable with protective ground, 4-pole, 2.5 mm², male GST18i4, female GST18i4, according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 BS 25 x0SW

Part No.: 92.208.x000.1 (x corresponds to length in m)

gesis® extension cable, female, male, 4x 2.5 mm², 250/400 V, 20 A, length: x m, pebble gray

System gesis® CON - assembled H05VV-F extension cable with protective ground, 4-pole, 2.5 mm², male GST18i4, female GST18i4, according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 BS 25 x0GR01

Part No.: 92.208.x000.3 (x corresponds to length in m)

gesis® connection cable, female, free end, 4x 2.5 mm², 250/400 V, 20 A, length: x m, white

System gesis® CON - assembled H05VV-F connection cable with protective ground, 4-pole, 2.5 mm², female GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 B- 25 x0WS

Part No.: 92.208.x003.2 (x corresponds to length in m)

gesis® connection cable, female, free end, 4x 2.5 mm², 250/400 V, 20 A, length: x m, black

System gesis® CON - assembled H05VV-F connection cable with protective ground, 4-pole, 2.5 mm², female GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 B- 25 x0SW

Part No.: 92.208.x003.1 (x corresponds to length in m)

gesis® connection cable, female, free end, 4x 2.5 mm², 250/400 V, 20 A, length: x m, pebble gray

System gesis® CON - assembled H05VV-F connection cable with protective ground, 4-pole, 2.5 mm², female GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 B- 25 x0GR01

Part No.: 92.208.x003.3 (x corresponds to length in m)

gesis® connection cable, male, free end, 4x 2.5 mm², 250/400 V, 20 A, length: x m, white

System gesis® CON - assembled H05VV-F connection cable with protective ground, 4-pole, 2.5 mm², male GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 -S 25 x0WS

Part No.: 92.208.x004.2 (x corresponds to length in m)

gesis® connection cable, male, free end, 4x 2.5 mm², 250/400 V, 20 A, length: x m, black

System gesis® CON - assembled H05VV-F connection cable with protective ground, 4-pole, 2.5 mm², male GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 -S 25 x0SW

Part No.: 92.208.x004.1 (x corresponds to length in m)

gesis® connection cable, male, free end, 4x 2.5 mm², 250/400 V, 20 A, length: x m, pebble gray

System gesis® CON - assembled H05VV-F connection cable with protective ground, 4-pole, 2.5 mm², male GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 -S 25 x0GR01

Part No.: 92.208.x004.3 (x corresponds to length in m)

      - Cable assemblies, 2.5 mm², halogen-free

gesis® extension cable, female, male, 4x 2.5 mm², 250/400 V, 20 A, length: x m, white

System gesis® CON - assembled H05Z1Z1-F extension cable with protective ground, 4-pole, 2.5 mm², male GST18i4, female GST18i4, according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: white

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 BS 25H x0WS

Part No.: 92.208.x060.1 (x corresponds to length in m)

gesis® extension cable, female, male, 4x 2.5 mm², 250/400 V, 20 A, length: x m, black

System gesis® CON - assembled H05Z1Z1-F extension cable with protective ground, 4-pole, 2.5 mm², male GST18i4, female GST18i4, according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 BS 25H x0SW

Part No.: 92.208.x060.2 (x corresponds to length in m)

gesis® extension cable, female, male, 4x 2.5 mm², 250/400 V, 20 A, length: x m, pebble gray

System gesis® CON - assembled H05Z1Z1-F extension cable with protective ground, 4-pole, 2.5 mm², male GST18i4, female GST18i4, according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 BS 25H x0GR01

Part No.: 92.208.x060.3 (x corresponds to length in m)

gesis® connection cable, female, free end, 4x 2.5 mm², 250/400 V, 20 A, length: x m, white

System gesis® CON - assembled H05Z1Z1-F connection cable with protective ground, 4-pole, 2.5 mm², female GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: white

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 B- 25H x0WS

Part No.: 92.208.x063.2 (x corresponds to length in m)

gesis® connection cable, female, free end, 4x 2.5 mm², 250/400 V, 20 A, length: x m, black

System gesis® CON - assembled H05Z1Z1-F connection cable with protective ground, 4-pole, 2.5 mm², female GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 B- 25H x0SW

Part No.: 92.208.x063.1 (x corresponds to length in m)

gesis® connection cable, female, free end, 4x 2.5 mm², 250/400 V, 20 A, length: x m, pebble gray

System gesis® CON - assembled H05Z1Z1-F connection cable with protective ground, 4-pole, 2.5 mm², female GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 B- 25H x0GR01

Part No.: 92.208.x063.3 (x corresponds to length in m)

gesis® connection cable, male, free end, 4x 2.5 mm², 250/400 V, 20 A, length: x m, white

System gesis® CON - assembled H05Z1Z1-F connection cable with protective ground, 4-pole, 2.5 mm², male GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: white

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 -S 25H x0WS

Part No.: 92.208.x064.2 (x corresponds to length in m)

gesis® connection cable, male, free end, 4x 2.5 mm², 250/400 V, 20 A, length: x m, black

System gesis® CON - assembled H05Z1Z1-F connection cable with protective ground, 4-pole, 2.5 mm², male GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 -S 25H x0SW

Part No.: 92.208.x064.1 (x corresponds to length in m)

gesis® connection cable, male, free end, 4x 2.5 mm², 250/400 V, 20 A, length: x m, pebble gray

System gesis® CON - assembled H05Z1Z1-F connection cable with protective ground, 4-pole, 2.5 mm², male GST18i4, free end (ultrasonic welded), according IEC 61535, mechanical coding code 2, color coding pebble gray.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i4 K1 -S 25H x0GR01

Part No.: 92.208.x064.3 (x corresponds to length in m)

  - 5-pole - GST18i5

    - Connectors

      - Female connector with screw connection

gesis® connector, female, 5-pole, 250/400 V, 20 A, white

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 5-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 8.5 - 12.5 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: 3, N, PE, 2, 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 S B1 Z R1 WS

Part No.: 92.953.4053.0

gesis® connector, female, 5-pole, 250/400 V, 20 A, black

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 5-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 8.5 - 12.5 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: 3, N, PE, 2, 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 S B1 Z R1 SW

Part No.: 92.953.4053.1

gesis® connector, female, 5-pole, 250/400 V, 20 A, pebble gray

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 5-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 3, color coding pebble gray.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 8.5 - 12.5 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: 3, N, PE, 2, 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 S B1 Z R1 GR01

Part No.: 92.953.4553.0

gesis® connector, female, 5-pole, 250 V, 20 A, pastel blue

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, for dimming applications, female version, 5-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 2, color coding pastel blue.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 8.5 - 12.5 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: L, N, PE, D2, D1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 S B1 Z R1 PB02

Part No.: 92.953.4453.0

gesis® connector, female, 5-pole, 50 V, 6 A, light blue

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, for low voltage applications, female version, 5-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 4, color coding light blue.

Rated voltage: 50 V

Rated current: 6 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 1.0 mm²

Terminal cross-section, stranded: 0.75 - 1.0 mm²

Cable diameter: 7.0 - 10.3 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: 5, 4, 3, 2, 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 S B1B Z R2 BL01

Part No.: 92.953.3353.0

      - Male with screw connection

gesis® connector, male, 5-pole, 250/400 V, 20 A, white

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 5-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 8.5 - 12.5 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: 1, 2, PE, N, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 S S1 Z R1 WS

Part No.: 92.954.4053.0

gesis® connector, male, 5-pole, 250/400 V, 20 A, black

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 5-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 8.5 - 12.5 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: 1, 2, PE, N, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 S S1 Z R1 SW

Part No.: 92.954.4053.1

gesis® connector, male, 5-pole, 250/400 V, 20 A, pebble gray

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 5-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 3, color coding pebble gray.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 8.5 - 12.5 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: 1, 2, PE, N, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 S S1 Z R1 GR01

Part No.: 92.954.4553.0

gesis® connector, male, 5-pole, 250 V, 20 A, pastel blue

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, for dimming applications, male version, 5-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 2, color coding pastel blue.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 8.5 - 12.5 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: D1, D2, PE, N, L

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 S S1 Z R1 PB02

Part No.: 92.954.4453.0

gesis® connector, male, 5-pole, 50 V, 6 A, light blue

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, for low voltage applications, male version, 5-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 4, color coding light blue.

Rated voltage: 50 V

Rated current: 6 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 0.75 - 1.0 mm²

Terminal cross-section, stranded: 0.75 - 1.0 mm²

Cable diameter: 7.0 - 10.3 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: 1, 2, 3, 4, 5

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 S S1B Z R2 BL01

Part No.: 92.954.3353.0

    - Snap-In

      - Female connector with spring clamp connection

gesis® Snap-In, female, 5-pole, for panel mounting, 250/400 V, 20 A, white

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 5-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: 3, N, PE, 2, 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 F B2 RV WS

Part No.: 92.051.9658.0

gesis® Snap-In, female, 5-pole, for panel mounting, 250/400 V, 20 A, black

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 5-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: 3, N, PE, 2, 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 F B2 RV SW

Part No.: 92.051.9658.1

gesis® Snap-In, female, 5-pole, for panel mounting, 250/400 V, 20 A, pebble gray

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, female version, 5-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 3, color coding pebble gray.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: 3, N, PE, 2, 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 F B2 RV GR01

Part No.: 92.051.9758.0

gesis® Snap-In, female, 5-pole, for panel mounting, 250 V, 20 A, pastel blue

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, for dimming applications, female version, 5-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 2, color coding pastel blue.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: L, N, PE, D2, D1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 F B2 RV PB02

Part No.: 92.051.8658.0

gesis® Snap-In, female, 5-pole, for panel mounting, 50 V, 6 A, light blue

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, for low voltage applications, female version, 5-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 4, color coding light blue.

Rated voltage: 50 V

Rated current: 6 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: 5, 4, 3, 2, 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 F B2 RV BL01

Part No.: 92.051.8358.0

      - Male with spring clamp connection

gesis® Snap-In, male, 5-pole, for panel mounting, 250/400 V, 20 A, white

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 5-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: 1, 2, PE, N, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 F S2 RV WS

Part No.: 92.052.9658.0

gesis® Snap-In, male, 5-pole, for panel mounting, 250/400 V, 20 A, black

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 5-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: 1, 2, PE, N, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 F S2 RV SW

Part No.: 92.052.9658.1

gesis® Snap-In, male, 5-pole, for panel mounting, 250/400 V, 20 A, pebble gray

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, male version, 5-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 3, color coding pebble gray.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: 1, 2, PE, N, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 F S2 RV GR01

Part No.: 92.052.9758.0

gesis® Snap-In, male, 5-pole, for panel mounting, 250 V, 20 A, pastel blue

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, for dimming applications, male version, 5-pole, turned contacts with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 2, color coding pastel blue.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Material thickness: 0.5 - 1.5 mm

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, stranded: 0.5 - 1.5 mm²

Housing material: Polyamide

Connections per pole: 2

Marking: D1, D2, PE, N, L

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 F S2 RV PB02

Part No.: 92.052.8658.0

    - Luminaire connection

gesis® pluggable luminaire connection adapter, 5-pole, 250 V, 20 A, pastel blue

System gesis® CON - gesis TOP adapter for direct, pluggable connection of corresponding assembled luminaires according to IEC 61535 for permanent connection, for dimming applications, 5-pole, turned contacts, with integrated locking, mechanical coding code 2, color coding pastel blue.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: IP20

Housing material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 V N 2P1 V PB02

Part No.: 92.050.2853.0

gesis® pluggable luminaire connection adapter, 5-pole, 250 V, 20 A, pastel blue

System gesis® CON - gesis LINECT adapter for direct, pluggable connection of corresponding assembled luminaires according to IEC 61535 for permanent connection, for dimming applications, 5-pole, turned contacts, with integrated locking, mechanical coding code 2, color coding pastel blue.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: IP20

Housing material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 V N 2P1 N V WS

Part No.: 92.050.2453.0

    - Distribution blocks

gesis® distribution block, 5-pole, 1 input, 2 outputs, 250/400 V, 20 A, black

System gesis® CON - Distribution block for installation connectors according to IEC 61535, T-shaped distribution block, 5-pole, turned contacts, 1 input, 2 outputs, with integrated locking on one side, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 V 2P1 T E SW

Part No.: 99.470.5772.2

gesis® distribution block, 5-pole, 1 input, 2 outputs, 250/400 V, 20 A, white

System gesis® CON - Distribution block for installation connectors according to IEC 61535, T-shaped distribution block, 5-pole, turned contacts, 1 input, 2 outputs, with integrated locking on two sides, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 V 2P1 T V WS

Part No.: 92.050.1053.0

gesis® distribution block, 5-pole, 1 input, 2 outputs, 250/400 V, 20 A, black

System gesis® CON - Distribution block for installation connectors according to IEC 61535, T-shaped distribution block, 5-pole, turned contacts, 1 input, 2 outputs, with integrated locking on two sides, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 V 2P1 T V SW

Part No.: 92.050.1053.1

gesis® distribution block, 5-pole, 1 input, 2 outputs, 250 V, 20 A, pastel blue

System gesis® CON - Distribution block for installation connectors according to IEC 61535, for dimming applications, T-shaped distribution block, 5-pole, turned contacts, 1 input, 2 outputs, with integrated locking on two sides, mechanical coding code 2, color coding pastel blue.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 V 2P1 T V PB02

Part No.: 92.050.1453.1

gesis® distribution block, 5-/3-pole, 1 input, 2 outputs, 250/400 V, 20 A, black

System gesis® CON - Distribution block for installation connectors according to IEC 61535, T-shaped distribution block, 5-/3-pole, turned contacts, 1 input, 2 outputs, with integrated locking on one side, with single phase output L1, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 V 2A1 T E 1 SW

Part No.: 99.420.5772.2

gesis® distribution block, 5-/3-pole, 1 input, 2 outputs, 250/400 V, 20 A, black

System gesis® CON - Distribution block for installation connectors according to IEC 61535, T-shaped distribution block, 5-/3-pole, turned contacts, 1 input, 2 outputs, with integrated locking on one side, with single phase output L2, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 V 2A1 T E 2 SW

Part No.: 99.421.5772.2

gesis® distribution block, 5-/3-pole, 1 input, 2 outputs, 250/400 V, 20 A, black

System gesis® CON - Distribution block for installation connectors according to IEC 61535, T-shaped distribution block, 5-/3-pole, turned contacts, 1 input, 2 outputs, with integrated locking on one side, with single phase output L3, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 V 2A1 T E 3 SW

Part No.: 99.422.5772.2

gesis® distribution block, 5-/3-pole, 1 input, 2 outputs, 250/400 V, 20 A, white

System gesis® CON - Distribution block for installation connectors according to IEC 61535, T-shaped distribution block, 5-pole, turned contacts, 1 input, 2 outputs, with integrated locking on two sides, with single phase output L1, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 V 2P1 T V 1 WS

Part No.: 92.050.4153.0

gesis® distribution block, 5-/3-pole, 1 input, 2 outputs, 250/400 V, 20 A, black

System gesis® CON - Distribution block for installation connectors according to IEC 61535, T-shaped distribution block, 5-pole, turned contacts, 1 input, 2 outputs, with integrated locking on two sides, with single phase output L1, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 V 2P1 T V 1 SW

Part No.: 92.050.4153.1

gesis® distribution block, 5-/3-pole, 1 input, 2 outputs, 250/400 V, 20 A, white

System gesis® CON - Distribution block for installation connectors according to IEC 61535, T-shaped distribution block, 5-pole, turned contacts, 1 input, 2 outputs, with integrated locking on two sides, with single phase output L2, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 V 2P1 T V 2 WS

Part No.: 92.050.4253.0

gesis® distribution block, 5-/3-pole, 1 input, 2 outputs, 250/400 V, 20 A, black

System gesis® CON - Distribution block for installation connectors according to IEC 61535, T-shaped distribution block, 5-pole, turned contacts, 1 input, 2 outputs, with integrated locking on two sides, with single phase output L2, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 V 2P1 T V 2 SW

Part No.: 92.050.4253.1

gesis® distribution block, 5-/3-pole, 1 input, 2 outputs, 250/400 V, 20 A, white

System gesis® CON - Distribution block for installation connectors according to IEC 61535, T-shaped distribution block, 5-pole, turned contacts, 1 input, 2 outputs, with integrated locking on two sides, with single phase output L3, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 V 2P1 T V 3 WS

Part No.: 92.050.4353.0

gesis® distribution block, 5-/3-pole, 1 input, 2 outputs, 250/400 V, 20 A, black

System gesis® CON - Distribution block for installation connectors according to IEC 61535, T-shaped distribution block, 5-pole, turned contacts, 1 input, 2 outputs, with integrated locking on two sides, with single phase output L3, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 V 2P1 T V 3 SW

Part No.: 92.050.4353.1

gesis® distribution block, 5-pole, 1 input, 3 outputs, 250/400 V, 20 A, white

System gesis® CON - Distribution block for installation connectors according to IEC 61535, H-shaped distribution block, 5-pole, turned contacts, 1 input, 3 outputs, with integrated locking, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 V 3P1 V WS

Part No.: 92.050.0353.0

gesis® distribution block, 5-pole, 1 input, 3 outputs, 250/400 V, 20 A, black

System gesis® CON - Distribution block for installation connectors according to IEC 61535, H-shaped distribution block, 5-pole, turned contacts, 1 input, 3 outputs, with integrated locking, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 V 3P1 V SW

Part No.: 92.050.0353.1

gesis® distribution block, 5-pole, 1 input, 3 outputs, 250/400 V, 20 A, pastel blue

System gesis® CON - Distribution block for installation connectors according to IEC 61535, for dimming applications, H-shaped distribution block, 5-pole, turned contacts, 1 input, 3 outputs, with integrated locking, mechanical coding code 2, color coding pastel blue.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 V 3P1 V PB02

Part No.: 92.050.1853.0

gesis® distribution block, 5-/3-pole, 1 input, 7 outputs, 250/400 V, 20 A, white

System gesis® CON - Distribution block for installation connectors according to IEC 61535, distribution block, 5-/3-pole, turned contacts, 1 input, 7 outputs, with integrated locking, with 1x forwarding 5-pole, 2x single phase output L1, 2x single phase output L2, 2x single phase output L3, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 7

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 V3 7A1 V WS

Part No.: 92.050.0153.0

gesis® distribution block, 5-/3-pole, 1 input, 7 outputs, 250/400 V, 20 A, black

System gesis® CON - Distribution block for installation connectors according to IEC 61535, distribution block, 5-/3-pole, turned contacts, 1 input, 7 outputs, with integrated locking, with 1x forwarding 5-pole, 2x single phase output L1, 2x single phase output L2, 2x single phase output L3, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Housing material: Polyamide

Input: 1

Outputs: 7

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 V3 7A1 V SW

Part No.: 92.050.0153.1

    - Accessories

gesis® locking GST18, 3-, 4- or 5-pole, white

System gesis® CON - locking for solder parts GST18, 3-, 4- or 5-pole, white.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18 WS

Part No.: 05.590.4556.0

gesis® locking GST18, 3-, 4- or 5-pole, black

System gesis® CON - locking for solder parts GST18, 3-, 4- or 5-pole, black.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18 SW

Part No.: 05.590.4556.1

gesis® locking GST18, 3-, 4- or 5-pole, white

System gesis® CON - locking for floating cable connections GST18, 3-, 4- or 5-pole, white.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18 WS

Part No.: 05.587.3156.0

gesis® locking GST18, 3-, 4- or 5-pole, black

System gesis® CON - locking for floating cable connections GST18, 3-, 4- or 5-pole, black.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18 SW

Part No.: 05.587.3156.1

gesis® mounting plate for distribution block 5-pole; 5-3-pole, black

System gesis® CON - mounting plate for distribution block 5-pole; 5-3-pole, black.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18 SW

Part No.: 07.413.5453.1

gesis® mounting sheet for distribution block 5-pole; 5-3-pole, black

System gesis® CON - mounting sheet for distribution block 5-pole; 5-3-pole, black.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18

Part No.: 05.562.3612.0

gesis® cover for female, 5-pole, black

System gesis® CON - Cover for female parts GST18i5, black.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 VS B SW

Part No.: 05.566.9253.0

gesis® cover for male, 5-pole, black

System gesis® CON - Cover for male parts GST18i5, black.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 VS S SW

Part No.: 05.583.1853.0

gesis® cover for female, 1-pole, white

System gesis® CON - Cover for female parts GST18, 3-, 4-, 5- or 6-pole, in a 12-pole pack, white.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18 WS

Part No.: 05.564.5353.2

gesis® cover for female, 1-pole, black

System gesis® CON - Cover for female parts GST18, 3-, 4-, 5- or 6-pole, in a 12-pole pack, black.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18 SW

Part No.: 05.564.5353.0

    - Cables

      - Cable assemblies, 1.5 mm², containing halogen

gesis® extension cable, female, male, 5x 1.5 mm², 250/400 V, 16 A, length: x m, white

System gesis® CON - assembled H05VV-F extension cable with protective ground, 5-pole, 1.5 mm², male GST18i5, female GST18i5, according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 BS 15 x0WS

Part No.: 92.257.x000.2 (x corresponds to length in m)

gesis® extension cable, female, male, 5x 1.5 mm², 250/400 V, 16 A, length: x m, black

System gesis® CON - assembled H05VV-F extension cable with protective ground, 5-pole, 1.5 mm², male GST18i5, female GST18i5, according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 BS 15 x0SW

Part No.: 92.257.x000.1 (x corresponds to length in m)

gesis® extension cable, female, male, 5x 1.5 mm², 250/400 V, 16 A, length: x m, pastel blue

System gesis® CON - assembled H05VV-F extension cable with protective ground, 5-pole, 1.5 mm², for dimming applications, male GST18i5, female GST18i5, according IEC 61535, mechanical coding code 2, color coding pastel blue.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 BS 15 x0PB02

Part No.: 92.257.x000.9 (x corresponds to length in m)

gesis® connection cable, female, free end, 5x 1.5 mm², 250/400 V, 16 A, length: x m, white

System gesis® CON - assembled H05VV-F connection cable with protective ground, 5-pole, 1.5 mm², female GST18i5, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 B- 15 x0WS

Part No.: 92.257.x003.2 (x corresponds to length in m)

gesis® connection cable, female, free end, 5x 1.5 mm², 250/400 V, 16 A, length: x m, black

System gesis® CON - assembled H05VV-F connection cable with protective ground, 5-pole, 1.5 mm², female GST18i5, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 B- 15 x0SW

Part No.: 92.257.x003.1 (x corresponds to length in m)

gesis® connection cable, female, free end, 5x 1.5 mm², 250 V, 16 A, length: x m, pastel blue

System gesis® CON - assembled H05VV-F connection cable with protective ground, 5-pole, 1.5 mm², for dimming applications, female GST18i5, free end (ultrasonic welded), according IEC 61535, mechanical coding code 2, color coding pastel blue.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 B- 15 x0PB02

Part No.: 92.257.x003.9 (x corresponds to length in m)

gesis® connection cable, male, free end, 5x 1.5 mm², 250/400 V, 16 A, length: x m, white

System gesis® CON - assembled H05VV-F connection cable with protective ground, 5-pole, 1.5 mm², male GST18i5, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 -S 15 x0WS

Part No.: 92.257.x004.2 (x corresponds to length in m)

gesis® connection cable, male, free end, 5x 1.5 mm², 250/400 V, 16 A, length: x m, black

System gesis® CON - assembled H05VV-F connection cable with protective ground, 5-pole, 1.5 mm², male GST18i5, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 -S 15 x0SW

Part No.: 92.257.x004.1 (x corresponds to length in m)

gesis® connection cable, male, free end, 5x 1.5 mm², 250 V, 16 A, length: x m, pastel blue

System gesis® CON - assembled H05VV-F connection cable with protective ground, 3-pole, 1.5 mm², for dimming applications, male GST18i5, free end (ultrasonic welded), according IEC 61535, mechanical coding code 2, color coding pastel blue.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 -S 15 x0PB02

Part No.: 92.257.x004.9 (x corresponds to length in m)

      - Cable assemblies, 1.5 mm², halogen-free

gesis® extension cable, female, male, 5x 1.5 mm², 250/400 V, 16 A, length: x m, white

System gesis® CON - assembled H05Z1Z1-F extension cable with protective ground, 5-pole, 1.5 mm², male GST18i5, female GST18i5, according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: white

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 BS 15H x0WS

Part No.: 92.257.x060.2 (x corresponds to length in m)

gesis® extension cable, female, male, 5x 1.5 mm², 250/400 V, 16 A, length: x m, black

System gesis® CON - assembled H05Z1Z1-F extension cable with protective ground, 5-pole, 1.5 mm², male GST18i5, female GST18i5, according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 BS 15H x0SW

Part No.: 92.257.x060.1 (x corresponds to length in m)

gesis® extension cable, female, male, 5x 1.5 mm², 250/400 V, 16 A, length: x m, pastel blue

System gesis® CON - assembled H05Z1Z1-F extension cable with protective ground, 5-pole, 1.5 mm², for dimming applications, male GST18i5, female GST18i5, according IEC 61535, mechanical coding code 2, color coding pastel blue.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 BS 15H x0PB02

Part No.: 92.257.x060.9 (x corresponds to length in m)

gesis® connection cable, female, free end, 5x 1.5 mm², 250/400 V, 16 A, length: x m, white

System gesis® CON - assembled H05Z1Z1-F connection cable with protective ground, 5-pole, 1.5 mm², female GST18i5, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: white

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 B- 15H x0WS

Part No.: 92.257.x063.2 (x corresponds to length in m)

gesis® connection cable, female, free end, 5x 1.5 mm², 250/400 V, 16 A, length: x m, black

System gesis® CON - assembled H05Z1Z1-F connection cable with protective ground, 5-pole, 1.5 mm², female GST18i5, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 B- 15H x0SW

Part No.: 92.257.x063.1 (x corresponds to length in m)

gesis® connection cable, female, free end, 5x 1.5 mm², 250 V, 16 A, length: x m, pastel blue

System gesis® CON - assembled H05Z1Z1-F connection cable with protective ground, 5-pole, 1.5 mm², for dimming applications, female GST18i5, free end (ultrasonic welded), according IEC 61535, mechanical coding code 2, color coding pastel blue.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 B- 15H x0PB02

Part No.: 92.257.x063.9 (x corresponds to length in m)

gesis® connection cable, male, free end, 5x 1.5 mm², 250/400 V, 16 A, length: x m, white

System gesis® CON - assembled H05Z1Z1-F connection cable with protective ground, 5-pole, 1.5 mm², male GST18i5, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: white

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 -S 15H x0WS

Part No.: 92.257.x064.2 (x corresponds to length in m)

gesis® connection cable, male, free end, 5x 1.5 mm², 250/400 V, 16 A, length: x m, black

System gesis® CON - assembled H05Z1Z1-F connection cable with protective ground, 5-pole, 1.5 mm², male GST18i5, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 -S 15H x0SW

Part No.: 92.257.x064.1 (x corresponds to length in m)

gesis® connection cable, male, free end, 5x 1.5 mm², 250 V, 16 A, length: x m, pastel blue

System gesis® CON - assembled H05Z1Z1-F connection cable with protective ground, 3-pole, 1.5 mm², for dimming applications, male GST18i5, free end (ultrasonic welded), according IEC 61535, mechanical coding code 2, color coding pastel blue.

Rated voltage: 250 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 -S 15H x0PB02

Part No.: 92.257.x064.9 (x corresponds to length in m)

      - Cable assemblies, 2.5 mm², containing halogen

gesis® extension cable, female, male, 5x 2.5 mm², 250/400 V, 20 A, length: x m, white

System gesis® CON - assembled H05VV-F extension cable with protective ground, 5-pole, 2.5 mm², male GST18i5, female GST18i5, according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 BS 25 x0WS

Part No.: 92.258.x000.2 (x corresponds to length in m)

gesis® extension cable, female, male, 5x 2.5 mm², 250/400 V, 20 A, length: x m, black

System gesis® CON - assembled H05VV-F extension cable with protective ground, 5-pole, 2.5 mm², male GST18i5, female GST18i5, according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 BS 25 x0SW

Part No.: 92.258.x000.1 (x corresponds to length in m)

gesis® connection cable, female, free end, 5x 2.5 mm², 250/400 V, 20 A, length: x m, white

System gesis® CON - assembled H05VV-F connection cable with protective ground, 5-pole, 2.5 mm², female GST18i5, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 B- 25 x0WS

Part No.: 92.258.x003.2 (x corresponds to length in m)

gesis® connection cable, female, free end, 5x 2.5 mm², 250/400 V, 20 A, length: x m, black

System gesis® CON - assembled H05VV-F connection cable with protective ground, 5-pole, 2.5 mm², female GST18i5, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 B- 25 x0SW

Part No.: 92.258.x003.1 (x corresponds to length in m)

gesis® connection cable, male, free end, 5x 2.5 mm², 250/400 V, 20 A, length: x m, white

System gesis® CON - assembled H05VV-F connection cable with protective ground, 5-pole, 2.5 mm², male GST18i5, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 -S 25 x0WS

Part No.: 92.258.x004.2 (x corresponds to length in m)

gesis® connection cable, male, free end, 5x 2.5 mm², 250/400 V, 20 A, length: x m, black

System gesis® CON - assembled H05VV-F connection cable with protective ground, 5-pole, 2.5 mm², male GST18i5, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05VV-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 -S 25 x0SW

Part No.: 92.258.x004.1 (x corresponds to length in m)

      - Cable assemblies, 2.5 mm², halogen-free

gesis® extension cable, female, male, 5x 2.5 mm², 250/400 V, 20 A, length: x m, white

System gesis® CON - assembled H05Z1Z1-F extension cable with protective ground, 5-pole, 2.5 mm², male GST18i5, female GST18i5, according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: white

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 BS 25H x0WS

Part No.: 92.258.x060.2 (x corresponds to length in m)

gesis® extension cable, female, male, 5x 2.5 mm², 250/400 V, 20 A, length: x m, black

System gesis® CON - assembled H05Z1Z1-F extension cable with protective ground, 5-pole, 2.5 mm², male GST18i5, female GST18i5, according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 BS 25H x0SW

Part No.: 92.258.x060.1 (x corresponds to length in m)

gesis® connection cable, female, free end, 5x 2.5 mm², 250/400 V, 20 A, length: x m, white

System gesis® CON - assembled H05Z1Z1-F connection cable with protective ground, 5-pole, 2.5 mm², female GST18i5, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: white

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 B- 25H x0WS

Part No.: 92.258.x063.2 (x corresponds to length in m)

gesis® connection cable, female, free end, 5x 2.5 mm², 250/400 V, 20 A, length: x m, black

System gesis® CON - assembled H05Z1Z1-F connection cable with protective ground, 5-pole, 2.5 mm², female GST18i5, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 B- 25H x0SW

Part No.: 92.258.x063.1 (x corresponds to length in m)

gesis® connection cable, male, free end, 5x 2.5 mm², 250/400 V, 20 A, length: x m, white

System gesis® CON - assembled H05Z1Z1-F connection cable with protective ground, 5-pole, 2.5 mm², male GST18i5, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding white.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: white

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 -S 25H x0WS

Part No.: 92.258.x064.2 (x corresponds to length in m)

gesis® connection cable, male, free end, 5x 2.5 mm², 250/400 V, 20 A, length: x m, black

System gesis® CON - assembled H05Z1Z1-F connection cable with protective ground, 5-pole, 2.5 mm², male GST18i5, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: H05Z1Z1-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i5 K1 -S 25H x0SW

Part No.: 92.258.x064.1 (x corresponds to length in m)

  - 6-pole - GST18i6

    - Connectors

      - Female connector with screw connection

gesis® connector, female, 6-pole, 250/400 V, 20 A, pastel blue/black

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, for dimming applications, 2-fold female, 2x 3-pole, turned contacts with screw connection, housing with strain relief, field assembly, mechanical coding code 1, color coding pastel blue/black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 1.5 mm²

Terminal cross-section, stranded: 1.5 mm²

Cable diameter: 10 - 12 mm (2x, S05 HH-F6)

Housing material: Polyamide

Connections per pole: 1

Marking: 3, 2, 1, N, PE, L

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i6 S B1N Z R1 V SWPB

Part No.: 92.969.0158.1

      - Male with screw connection

gesis® connector, male, 6-pole, 250/400 V, 20 A, pastel blue/black

System gesis® CON - Installation connector according to IEC 61535 for permanent connection, for dimming applications, 2-fold male, 2x 3-pole, turned contacts with screw connection, housing with strain relief, field assembly, mechanical coding code 1, color coding pastel blue/black.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP40

Terminal cross-section, solid: 1.5 mm²

Terminal cross-section, stranded: 1.5 mm²

Cable diameter: 10 - 12 mm (2x, S05 HH-F6)

Housing material: Polyamide

Connections per pole: 1

Marking: L, N, PE, 1, 2, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i6 S S1N Z R1 V SWPB

Part No.: 92.968.0158.1

    - Accessories

gesis® cover for female, 1-pole, white

System gesis® CON - Cover for female parts GST18, 3-, 4-, 5- or 6-pole, in a 12-pole pack, white.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18 WS

Part No.: 05.564.5353.2

gesis® cover for female, 1-pole, black

System gesis® CON - Cover for female parts GST18, 3-, 4-, 5- or 6-pole, in a 12-pole pack, black.

Material: Polyamide

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18 SW

Part No.: 05.564.5353.0

    - Cables

      - Cable assemblies, 1.5 mm², halogen-free

gesis® extension cable, female, male, 6x 1.5 mm², 250/400 V, 16 A, length: x m, pastel blue/black

System gesis® CON - assembled S05HH-F extension cable with protective ground, 6-pole, 1.5 mm², for dimming applications, male GST18i6, female GST18i6, according IEC 61535, mechanical coding code 1, color coding pastel blue/black.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05HH-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i6 K1 BS 15H x0SW

Part No.: 99.41x.0028.0 (x corresponds to length in m)

gesis® connection cable, female, free end, 6x 1.5 mm², 250/400 V, 16 A, length: x m, pastel blue/black

System gesis® CON - assembled S05HH-F connection cable with protective ground, 6-pole, 1.5 mm², for dimming applications, female GST18i6, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding pastel blue/black.

Rated voltage: 250/400 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05HH-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i6 K1 B- 15H x0SW

Part No.: 99.43x.0028.0 (x corresponds to length in m)

gesis® connection cable, male, free end, 6x 1.5 mm², 250/400 V, 16 A, length: x m, pastel blue/black

System gesis® CON - assembled S05HH-F connection cable with protective ground, 6-pole, 1.5 mm², for dimming applications, male GST18i6, free end (ultrasonic welded), according IEC 61535, mechanical coding code 1, color coding pastel blue/black.

Rated voltage: 250/600 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection (plugged): IP40

Type of protection (unplugged): IP20

Cable type: S05HH-F

Cable color: black

Halogen-free: yes

Length: x m

Brand: Wieland Electric GmbH

Type: GST18i6 K1 -S 15H x0SW

Part No.: 99.45x.0028.0 (x corresponds to length in m)

05.561.3653.0	27

05.561.3653.1	27

05.562.3612.0	104

05.564.5353.0	28, 71, 104, 120

05.564.5353.2	28, 71, 104, 120

05.566.9153.0	71

05.566.9253.0	104

05.583.1853.0	104

05.587.3156.0	26, 70, 103

05.587.3156.1	27, 71, 103

05.590.4556.0	26, 70, 103

05.590.4556.1	26, 70, 103

05.594.4250.0	27

05.594.4253.0	27

07.413.5453.1	103

92.030.1053.0	19

92.030.1053.1	19

92.030.1253.0	20

92.030.1253.1	20

92.030.1353.0	20

92.030.2153.0	18

92.030.2253.0	18

92.030.4853.0	21

92.030.4853.1	22

92.030.4953.0	22

92.030.4953.1	22

92.030.5253.0	23

92.030.5253.1	24

92.030.5353.0	23

92.030.5353.1	23

92.030.5853.0	26

92.030.5958.0	25

92.030.5958.1	25

92.030.6053.0	21

92.030.6053.1	21

92.031.7658.0	11

92.031.7658.1	12

92.031.8458.0	13

92.031.8558.0	13

92.031.8758.0	10

92.031.8758.1	10

92.031.9658.0	11

92.031.9658.1	12

92.031.9758.0	12

92.032.7658.0	15

92.032.7658.1	16

92.032.8458.0	17

92.032.8558.0	17

92.032.8758.0	14

92.032.8758.1	14

92.032.9658.0	15

92.032.9658.1	16

92.032.9758.0	16

92.040.1053.0	68

92.040.1053.1	68

92.040.1153.0	69

92.041.9658.0	65

92.041.9658.1	65

92.041.9758.0	66

92.042.9658.0	67

92.042.9658.1	67

92.050.0153.0	101

92.050.0153.1	102

92.050.0353.0	100

92.050.0353.1	101

92.050.1053.0	96

92.050.1053.1	97

92.050.1453.1	97

92.050.1853.0	101

92.050.2453.0	95

92.050.2853.0	95

92.050.4153.0	98

92.050.4153.1	99

92.050.4253.0	99

92.050.4253.1	99

92.050.4353.0	100

92.050.4353.1	100

92.051.8358.0	92

92.051.8658.0	92

92.051.9658.0	91

92.051.9658.1	91

92.051.9758.0	92

92.052.8658.0	94

92.052.9658.0	93

92.052.9658.1	93

92.052.9758.0	94

92.207.x000.1	72

92.207.x000.2	72

92.207.x000.3	73

92.207.x003.1	73

92.207.x003.2	73

92.207.x003.3	73

92.207.x004.1	74

92.207.x004.2	74

92.207.x004.3	74

92.207.x060.1	76

92.207.x060.2	76

92.207.x060.3	76

92.207.x063.1	77

92.207.x063.2	77

92.207.x063.3	77

92.207.x064.1	78

92.207.x064.2	78

92.207.x064.3	78

92.208.x000.1	80

92.208.x000.2	80

92.208.x000.3	80

92.208.x003.1	81

92.208.x003.2	81

92.208.x003.3	81

92.208.x004.1	82

92.208.x004.2	82

92.208.x004.3	82

92.208.x060.1	83

92.208.x060.2	83

92.208.x060.3	83

92.208.x063.1	84

92.208.x063.2	84

92.208.x063.3	84

92.208.x064.1	85

92.208.x064.2	85

92.208.x064.3	85

92.232.x000.1	29

92.232.x000.2	29

92.232.x000.3	30

92.232.x000.5	30

92.232.x003.1	30

92.232.x003.2	30

92.232.x003.3	31

92.232.x003.5	31

92.232.x004.1	32

92.232.x004.2	31

92.232.x004.3	32

92.232.x004.5	32

92.232.x007.1	56

92.232.x007.2	56

92.232.x010.1	39

92.232.x010.2	39

92.232.x013.1	40

92.232.x013.2	40

92.232.x014.1	41

92.232.x014.2	40

92.232.x060.1	34

92.232.x060.2	34

92.232.x060.3	34

92.232.x060.5	35

92.232.x063.1	35

92.232.x063.2	35

92.232.x063.3	36

92.232.x063.5	36

92.232.x064.1	37

92.232.x064.2	36

92.232.x064.3	37

92.232.x064.5	37

92.238.x000.1	42

92.238.x000.2	42

92.238.x000.3	42

92.238.x000.5	43

92.238.x003.1	43

92.238.x003.2	43

92.238.x003.3	44

92.238.x003.5	44

92.238.x004.1	45

92.238.x004.2	44

92.238.x004.3	45

92.238.x004.5	45

92.238.x010.1	51

92.238.x010.2	51

92.238.x013.1	52

92.238.x013.2	52

92.238.x014.1	53

92.238.x014.2	52

92.238.x060.1	46

92.238.x060.2	46

92.238.x060.3	46

92.238.x060.5	47

92.238.x063.1	47

92.238.x063.2	47

92.238.x063.3	48

92.238.x063.5	48

92.238.x064.1	49

92.238.x064.2	48

92.238.x064.3	49

92.238.x064.5	49

92.257.x000.1	105

92.257.x000.2	105

92.257.x000.9	106

92.257.x003.1	106

92.257.x003.2	106

92.257.x003.9	107

92.257.x004.1	107

92.257.x004.2	107

92.257.x004.9	107

92.257.x060.1	109

92.257.x060.2	109

92.257.x060.9	109

92.257.x063.1	110

92.257.x063.2	110

92.257.x063.9	111

92.257.x064.1	111

92.257.x064.2	111

92.257.x064.9	111

92.258.x000.1	113

92.258.x000.2	113

92.258.x003.1	114

92.258.x003.2	113

92.258.x004.1	114

92.258.x004.2	114

92.258.x060.1	115

92.258.x060.2	115

92.258.x063.1	116

92.258.x063.2	115

92.258.x064.1	116

92.258.x064.2	116

92.731.6053.0	24

92.931.3053.0	2

92.931.3053.1	2

92.931.3453.0	3

92.931.3653.0	3

92.931.3853.0	3

92.932.3053.0	6

92.932.3053.1	6

92.932.3453.0	7

92.932.3653.0	7

92.932.3853.0	7

92.933.0053.0	4

92.933.0053.1	4

92.933.0153.0	5

92.933.0453.0	5

92.933.0553.0	5

92.934.0053.0	8

92.934.0053.1	8

92.934.0153.0	9

92.934.0453.0	9

92.934.0553.0	9

92.943.0053.0	59

92.943.0053.1	59

92.943.0153.0	60

92.943.3053.0	57

92.943.3053.1	57

92.943.3553.0	58

92.944.0053.0	63

92.944.0053.1	63

92.944.0153.0	64

92.944.3053.0	61

92.944.3053.1	61

92.944.3553.0	62

92.953.3353.0	88

92.953.4053.0	87

92.953.4053.1	87

92.953.4453.0	88

92.953.4553.0	88

92.954.3353.0	90

92.954.4053.0	89

92.954.4053.1	89

92.954.4453.0	90

92.954.4553.0	90

92.968.0158.1	119

92.969.0158.1	118

99.400.5623.1	56

99.41x.0028.0	121

99.420.5772.2	97

99.421.5772.2	98

99.422.5772.2	98

99.43x.0028.0	121

99.452.9910.0	25

99.453.9910.0	70

99.45x.0028.0	122

99.470.5772.2	96

99.600.0028.0	54

99.601.0028.0	55

99.602.0028.0	54

99.612.0028.0	54

99.652.0028.0	55

99.653.0028.0	55