Installations for WIN7:

1. you have to download a new firmware to the plotter with a WIN XP System:

Please select for wieplot500 A3 the directory wieplot_500_A3_1-16 :

Then start the file "update wieplot_500_A3_1-16.bat" and select the USB-Interface.

Please select for wieplot500 A4 the directory wieplot_500_A4_1-16:

Then start the file "update wieplot_500_A4_1-16.bat" and select the USB-Interface.

Please select for wieplotbasic the directory wieplot_basic_1-16:

Then start the file "update wieplot_basic_1-16.bat" and select the USB-Interface.

In order to do so, turn off the plotter,

hold down the button Clear buffer and the button STOP together and press ON again.

The control LEDs STOP and Clear buffer are on.

Confirm the setting with the SEND button and the update process

starts.The plotter will turn off automatically after completion.

2. you can install the new wiemarc software and the driver as descibed in the file "Installing_the_plotter_driver_in_Windows_7.pdf".